Another Look 
at Freezing Temperature

In the experiment, “Freezing and Melting of Water,” you saw that the temperature of pure water remained constant at its freezing temperature as it froze and melted. You will now observe what happens when a different pure substance, phenyl salicylate, freezes. Using a second Temperature Probe and sample, you will also see the effect on the freezing temperature when a small amount of another substance, benzoic acid, is dissolved in the phenyl salicylate.

MATERIALS

| LabPro or CBL 2 interface | 2 utility clamps |
| --- | --- |
| TI Graphing Calculator | test tube with phenyl salicylate |
| DataMate program | test tube with phenyl salicylate & benzoic acid |
| 2 Temperature Probes | 2 crystals of solid phenyl salicylate |
| 1-liter beaker | thermometer (optional) |
| ring stand |  |

PROCEDURE

1.	Obtain and wear goggles.

2.	Connect one Temperature Probe into Channel 1 and another Temperature Probe into Channel 2 of the LabPro or CBL 2 interface. Use the link cable to connect the TI Graphing Calculator to the interface. Firmly press in the cable ends.

3.	Fill a 1-liter beaker about 4/5 full with water at a temperature in the range 30-32°C. This temperature can be obtained by blending hot and cold tap water. This water temperature should be measured with a thermometer or Temperature Probe. Place the 1-liter beaker on the base of the ring stand.

Figure 1

4.	Obtain 2 crystals of solid phenyl salicylate to use in Step 12.

5.	Turn on the calculator and start the DATAMATE program. Press  to reset the program.

6.	Set up the calculator and interface for two Temperature Probes and two temperature calibrations.

- Select SETUP from the main screen.

- If the calculator displays two Temperature Probes, one in CH 1 and another in CH 2, proceed directly to Step 7. If it does not, continue with this step to set up your sensor manually.

- Press  to select CH 1.

- Select TEMPERATURE from the SELECT SENSOR menu.

- Select the Temperature Probe you are using (in °C) from the TEMPERATURE menu.

- Press  once, then press  to select CH2.

- Select TEMPERATURE from the SELECT SENSOR menu.

- Select the Temperature Probe you are using (in °C) from the TEMPERATURE menu.

7.	Set up the data-collection mode.

- To select MODE, use  to move the cursor to MODE, and press .

- Select TIME GRAPH from the SELECT MODE menu.

- Select CHANGE TIME SETTINGS from the TIME GRAPH SETTINGS menu.

- Enter “10” as the time between samples in seconds.

- Enter “90” as the number of samples. (The length of the data collection will be 15 minutes.)

- Select OK to return to the setup screen.

- Select OK again to return to the main screen.

8.	Identify each of the Temperature Probes.

- Probe 1 will monitor the temperature of phenyl salicylate and be connected to Channel 1.

- Probe 2 will monitor the temperature of the benzoic acid-phenyl salicylate mixture and be connected to Channel 2.

Important: Plan ahead for quick positioning of the probes and test tubes in Step 10!

9.	When everything is ready, use two utility clamps to obtain test tubes containing the two hot liquids from your teacher. While one team member obtains the melted phenyl salicylate, another can get the melted benzoic acid-phenyl salicylate mixture. Fasten the utility clamps at the top of the test tubes to carry them back to your lab station. CAUTION: Be careful not to spill the hot liquids and do not touch the bottom of the test tubes.

10.	Immediately insert Probe 1 into the hot phenyl salicylate and Probe 2 into the hot benzoic acid-phenyl salicylate mixture. Then attach the clamps to the ring stand with the test tubes still above the water. About 30 seconds are required for the probes to warm up to the temperature of their surroundings and give correct temperature readings.

After the 30 seconds have elapsed, select START to begin collecting temperature data. A live graph of temperature vs. time for both Probe 1 and Probe 2 is being plotted on the calculator screen. The live readings are displayed in the upper-right corner of the graph, Probe 1 first, Probe 2 below.

11.	Lower one test tube, then the other, into the water bath as shown in Figure 1. Make sure the water level outside the test tubes is higher than the liquid levels inside the test tubes. Probe 1 is now monitoring the phenyl salicylate and Probe 2 is monitoring the benzoic acid-phenyl salicylate mixture. Important: If the temperature at the start of the graph was not above 50°C for either one of your Temperature Probes, obtain two more samples and begin again.

12.	With a very slight up and down motion of both probes, continuously stir the liquids during the cooling. Hold the tops of the probes and not their wires! If no solid appears in either test tube by the time the temperature drops below 39°C, add a crystal of solid phenyl salicylate to the liquid.

13.	When either probe begins to stick in the phenyl salicylate, stop stirring. Continue on with the experiment until data collection stops after 15 minutes (or press the  key to stop before 15 minutes has elapsed). Disconnect the Temperature Probes from the interface and set the interface and calculator aside while you finish Steps 14 and 15.

14.	After collecting data, if a Temperature Probe has frozen into the phenyl salicylate, use the hot water bath provided by your teacher to melt it out. Do not attempt to pull the probe out—this might damage it!

15.	Return the test tubes containing the phenyl salicylate and the benzoic acid-phenyl salicylate mixture to the places directed by your instructor. Proceed directly to Step 1 of Processing the Data.

Processing the data

1.	On the displayed graph of temperature vs. time, each point for Probe 1 is plotted with a dot, and each point for Probe 2 with a box. As you move the cursor right or left, the time (X) and temperature (Y) values of each Probe 1 data point are displayed below the graph. Based on your data, what was the freezing temperature of pure phenyl salicylate (round to the nearest 0.1°C)?

2.	Press  to switch the cursor to the curve of temperature vs. time for Probe 2. Examine the data points along the curve. Record t2 and t1 for Probe 2. As you move the cursor right or left along this line, its time and temperature values are now displayed below the graph. Based on your data, what was the initial freezing temperature of the benzoic acid-phenyl salicylate mixture (round to the nearest 0.1°C)? (optional) Print a copy of the graph. Label the two curves on the graph as pure phenyl salicylate or benzoic acid-phenyl salicylate mixture.

3.	How are the curves on the two graphs different? You should see at least two effects that dissolving benzoic acid in phenyl salicylate had on the freezing process.

4.	Based on this lab, what method could a chemist use to determine whether an unknown liquid is pure or has an impurity dissolved in it?

DATA TABLE

| Melting temperature, phenyl salicylate | °C |
| --- | --- |
| Melting temperature, benzoic acid-phenyl salicylate mixture | °C |