Determining the Quantity
of Iron in a Vitamin Tablet

As biochemical research becomes more sophisticated, we are learning more about the role of metallic elements in the human body. For example, copper and zinc are present in enzymes, and trace amounts of molybdenum and selenium are vital in regulating internal oxidation-reduction reactions. Iron is necessary for oxygen transport in the bloodstream. Many people gain these essential elements through their diets, or by taking multivitamin tablets.

The iron that is present in these tablets is in the form of water-soluble Fe2+ ions. In this experiment, you will be using the Colorimeter to determine the quantity of iron in a vitamin tablet. You will prepare four solutions of known Fe2+ concentration. The Colorimeter will be used to determine the absorbance of each solution. When a graph of absorbance vs. concentration is plotted for these solutions, a direct relationship should result (Beer’s law).

The Colorimeter is also used to measure the absorbance of a solution prepared from a multivitamin tablet. This absorbance value is converted to concentration using the Beer’s law curve. From this concentration, the total mass of iron in the original tablet can be calculated.

The iron in each tablet tested will be reacted with 1,10 phenanthroline so that it develops a color that the Colorimeter can detect. Sodium acetate and hydroxylamine hydrochloride are added to control the pH of the mixtures and keep the iron in the +2 oxidation state.

MATERIALS

| LabPro or CBL 2 interface | multivitamin tablet with iron |
| --- | --- |
| TI Graphing Calculator | 20 mg/L iron (II) solution |
| DataMate program | 1% hydroxylamine hydrochloride |
| Vernier Colorimeter | 1 M sodium acetate |
| one cuvette | 1% 1,10 phenanthroline |
| four 20 X 150 mm test tubes | 0.010 M HCl |
| test tube rack | distilled water |
| mortar and pestle | 50-mL graduated cylinder |
| hot plate | two 100-mL beakers |
| filter funnel | two 10-mL graduated cylinders |
| filter paper | pipet pump or pipet bulb |
| stirring rod | two 10-mL pipets |

PROCEDURE

1.	Obtain and wear goggles.

2.	Weigh one multivitamin tablet and record its mass to the nearest 0.001 g. Use a mortar and pestle to grind the tablet to a fine powder. Also record the mass of iron, in mg, listed on the label of the vitamin container.

3.	Measure out 0.200 g of the powdered tablet and transfer it to a 100-mL beaker. Note: Try to avoid including bits of the tablet coating in your 0.200 g sample. Add 20 mL of 0.010 M HCl and 10 mL of distilled water. Mix thoroughly. CAUTION: Handle the hydrochloric acid with care. It can cause painful burns if it comes in contact with the skin.

4.	Use a hot plate to heat the mixture in the beaker to a gentle boil. Boil the mixture for 5-7 minutes. Frequently stir the mixture with a stirring rod. After boiling, allow the mixture to cool to nearly room temperature. Some particles may remain undissolved, which is normal and will not affect your experiment.

5.	Set up a funnel and filter paper. Pour the mixture through the filter, collecting the liquid filtrate in a 50-mL graduated cylinder. Rinse the 100-mL beaker with 10 mL of distilled water and pour the water through the filter. When the filter has drained, fill the graduated cylinder to the 50-mL mark with distilled water. Transfer this solution to a clean, dry, 100-mL beaker.

6.	Label five clean, dry, test tubes 1-5. Prepare a set of four standards and one blank solution in the test tubes using the following solutions: CAUTION: Handle the solutions with care.

- 20 mg/L iron (II) solution

- 1% hydroxylamine hydrochloride

- 1 M sodium acetate

- 1% 1,10 phenanthroline

Note: Solutions B, C, D, and the distilled water may be measured successively into one 10-mL graduated cylinder. Then add solution A, using a pipet for greater precision. Transfer the solution into a test tube. Important: Mix each solution thoroughly with a stirring rod.

| Test Tube number | A (mL) | B (mL) | C (mL) | D (mL) | Distilled water (mL) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.00 | 1.00 | 1.00 | 1.00 | 6.00 |
| 2 | 2.00 | 1.00 | 1.00 | 1.00 | 5.00 |
| 3 | 3.00 | 1.00 | 1.00 | 1.00 | 4.00 |
| 4 | 4.00 | 1.00 | 1.00 | 1.00 | 3.00 |
| 5 (blank) | 0.00 | 1.00 | 1.00 | 1.00 | 7.00 |

7.	Plug the Colorimeter into Channel 1 of the LabPro or CBL 2 interface. Use the link cable to connect the TI Graphing Calculator to the interface. Firmly press in the cable ends.

8.	Prepare a blank by filling an empty cuvette 3/4 full with distilled water. Seal the cuvette with a lid. To correctly use a Colorimeter cuvette, remember:

- All cuvettes should be wiped clean and dry on the outside with a tissue.

- Handle cuvettes only by the top edge of the ribbed sides.

- All solutions should be free of bubbles.

- Always position the cuvette with its reference mark facing toward the white reference mark at the right of the cuvette slot on the Colorimeter.

9.	Turn on the calculator and start the DATAMATE program. Press  to reset the program.

10.	Set up the calculator and interface for the Colorimeter.

- Place the blank in the cuvette slot of the Colorimeter and close the lid.

- Select SETUP from the main screen.

- If the calculator displays COLORIMETER in CH 1, set the wavelength on the Colorimeter to 565 nm. Then calibrate by pressing the AUTO CAL button on the Colorimeter and proceed directly to Step 11. If the calculator does not display COLORIMETER in CH1, continue with this step to set up your sensor manually.

- Press  to select CH 1.

- Select COLORIMETER from the SELECT SENSOR menu.

- Select CALIBRATE from the SETUP menu.

- Select CALIBRATE NOW from the CALIBRATION menu.

First Calibration Point

- Turn the wavelength knob of the Colorimeter to the 0% T position. When the voltage reading stabilizes, press . Enter “0” as the percent transmittance.

Second Calibration Point

- Turn the wavelength knob of the Colorimeter to the Green LED position (565 nm). When the voltage reading stabilizes, press . Enter “100” as the percent transmittance.

- Select OK to return to the setup screen.

11.	Set up the data-collection mode.

- To select MODE, press  once and press .

- Select EVENTS WITH ENTRY from the SELECT MODE menu.

- Select OK to return to the main screen.

12.	You are now ready to collect absorbance-concentration data for the five standard solutions.

- Select START from the main screen.

- Empty the water from the cuvette. Using the solution in Test Tube 1, rinse the cuvette twice with ~1-mL amounts and then fill it 3/4 full. Wipe the outside with a tissue, place it in the Colorimeter, and close the lid.

- When the value displayed on the calculator screen has stabilized, press . Enter “2.00” as the iron concentration in mg/L. The absorbance and concentration values have now been saved for the first solution.

- Discard the cuvette contents as directed by your instructor. Using the solution in Test Tube 2, rinse the cuvette twice with ~1-mL amounts and then fill it 3/4 full. After closing the lid, wait for the value displayed on the calculator screen to stabilize and press . Enter “4.00” as the concentration in mg/L.

- Repeat the procedure for Test Tube 3 (6.00 mg/L) and Test Tube 4 (8.00 mg/L).

- Press  to stop data collection. The absorbance and concentration values have now been saved for the standard solutions.

- Examine the data points along the curve on the displayed graph. As you move the cursor right or left, the concentration (X) and absorbance (Y) values of each data point are displayed below the graph. Record the absorbance values in your data table (round to the nearest 0.001).

- Press  to return to the main screen.

13.	Prepare the tablet solution in the 100-mL beaker for testing.

- Measure out 5.00 mL of the tablet solution into a second clean, dry, 100-mL beaker.

- Add 5.00 mL each of solutions B, C, and D, plus 30.0 mL of distilled water.

- Mix the solution thoroughly.

14.	To find the absorbance of the tablet solution:

- Rinse the cuvette twice with the tablet solution you prepared in Step 13, and fill it about 3/4 full.

- Wipe the outside of the cuvette, place it into the Colorimeter, and close the lid. CAUTION: Handle the solutions with care.

- Monitor the absorbance value displayed on the calculator. When this value has stabilized, record the absorbance value for the tablet solution in your data table (round to the 
nearest 0.001).

- Dispose of the remaining solution in the flask as directed by your instructor.

15.	Discard the solutions as directed by your instructor. Proceed directly to Steps 1–4 of Processing the Data.

Processing the data

1.	To determine the iron concentration in tablet solution, you can use your TI calculator and interpolate along the regression line on your Beer’s law curve. Use the following method to convert the absorbance value of the unknown to concentration, in mg/L.

- Select ANALYZE from the main screen.

- Select CURVE FIT from the ANALYZE OPTIONS menu.

- Select LINEAR (CH 1 VS ENTRY) from the CURVE FIT menu. The linear-regression statistics for these two lists are displayed for the equation in the form

y = ax + b

- where y is absorbance, x is concentration, a is the slope, and b is the y-intercept. Note: One indicator of the quality of your data is the size of b. It is a very small value if the regression line passes through or near the origin. The correlation coefficient, r, indicates how closely the data points match up with (or fit) the regression line. A value of 1.00 indicates a nearly perfect fit.

- To display the linear-regression curve on the graph of absorbance vs. concentration, press . This graph should indicate a direct relationship between absorbance and concentration, a relationship known as Beer’s law. The regression line should closely fit the five data points and pass through (or near) the origin of the graph.

- To interpolate along the curve, press . A cursor is displayed on the regression line, along with its X and Y coordinates below the graph. Use  or  to move the cursor to an absorbance value (Y value) that is closest to the absorbance reading you obtained in Step 14. The iron concentration of your tablet solution, in mg/L, will be equal to the corresponding X value. Record this value in your data table.

2.	Print a graph of absorbance vs. concentration, with a regression line and interpolated unknown concentration displayed.

3.	Determine the mass of iron in your vitamin tablet:

- Because the tablet solution in Step 13 was diluted by a factor of 1/10, multiply your final iron concentration (in mg/L) by 10 to find the pre-dilution iron concentration, in mg/L.

- A portion of the tablet was dissolved in 50 mL of solution in Step 5. Use this volume and the answer in the previous step to calculate the mass of iron, in mg, in the tablet solution.

- Since the analysis used only a fraction of the total mass of the tablet (see Step 3), multiply the mg of iron in the previous step by the ratio of the total tablet mass to the 0.200 g portion:

mg of iron X = mg of iron in the vitamin tablet

4.	Compare your final answer in the previous step with the mg of iron shown on the label of vitamin tablets you tested. Use these values to calculate the percent error.

DATA and calculations

| Mass of tablet | g |
| --- | --- |
| Mass of iron on label | mg |

| Test Tube Number | Concentration (mg/L) | Absorbance |
| --- | --- | --- |
| 1 | 2.00 |  |
| 2 | 4.00 |  |
| 3 | 6.00 |  |
| 4 | 8.00 |  |
| tablet solution |  |  |

| Iron concentration in the pre-dilution tablet solution (use a factor of 10) | mg/L |
| --- | --- |
| Mass of iron in the original tablet solution | mg |
| Calculated mass of iron in vitamin tablet | mg |
| Percent error | % |