Heat of Fusion for Ice

Melting and freezing behavior are among the characteristic properties that give a pure substance its unique identity. As energy is added, pure solid water (ice) at 0°C changes to liquid water at 0°C.

In this experiment, you will determine the energy (in joules) required to melt one gram of ice. You will then determine the molar heat of fusion for ice (in kJ/mol). Excess ice will be added to warm water, at a known temperature, in a Styrofoam cup. The warm water will be cooled down to a temperature near 0°C by the ice. The energy required to melt the ice is removed from the warm water as it cools.

To calculate the heat that flows from the water, you can use the relationship

q = Cp•m•Dt

where q stands for heat flow, Cp is specific heat capacity, m is mass in grams, and Dt is the change in temperature. For water, Cp is 4.18 J/g°C.

MATERIALS

| TI-83 Plus or TI-84 Plus graphing calculator | Styrofoam cup |
| --- | --- |
| EasyData application | ring stand |
| Temperature Probe and data-collection interface | utility clamp |
| or EasyTemp | ice cubes |
| 250 mL beaker | stirring rod |
| 100 mL beaker | warm water |
| tongs |  |

PROCEDURE

Part I  Freezing

1.	Turn on the calculator. Connect the Temperature Probe, data-collection interface, and calculator. (If you are using an EasyTemp, you do not need a data-collection interface.)

2.	Use a utility clamp the Temperature Probe on a ring stand as shown in Figure 1.

3.	Place a Styrofoam cup into a 250 mL beaker as shown in Figure 1.

4.	Use a 100 mL graduated cylinder to obtain 100.0 mL of water at about 60°C from your teacher.

5.	Obtain 7 or 8 large ice cubes.

6.	Set up EasyData for data collection.

- Start EasyData if it is not already running.

- Select  from the Main screen, and then select New to reset the application.

- Select  from the Main screen, then select Time Graph…

- Select  on the Time Graph Settings screen.

- Enter 5 as the time between samples in seconds and select .

- Enter 96 as the number of samples and select  (data will be collected for 8 minutes).

- Select  to return to the Main screen.

7.	Lower the Temperature Probe into the warm water (to about 1 cm from the bottom).

8.	Select  to begin to monitor temperature. The temperature reading, in °C, is displayed on the upper-right corner of the graph. Wait until the temperature reaches a maximum (it will take a few seconds for the cold probe to reach the temperature of the warm water). This maximum will determine the initial temperature, t1, of the water. As soon as this maximum temperature is reached, fill the Styrofoam cup with ice cubes. Shake excess water from the ice cubes before adding them (or dry with a paper towel). Record the maximum temperature, t1, in your data table.

Figure 1

9.	Use a stirring rod to stir the mixture as the temperature approaches 0°C. Important: As the ice melts, add more large ice cubes to keep the mixture full of ice!

10.	When the temperature reaches about 4°C, quickly remove the unmelted ice (using tongs). Continue stirring until the temperature reaches a minimum (and begins to rise). This minimum temperature is the final temperature, t2, of the water. Record t2 in your data table.

11.	Data collection will stop after 8 minutes (or select  to stop before 8 minutes has elapsed). Use the 100 mL graduated cylinder to measure the volume of water remaining in the Styrofoam cup to the nearest 0.1 mL. Record this as V2.

12.	To confirm the t1 and t2 values you recorded earlier, examine the data points along the curve on the displayed graph. As you move the cursor right or left, the time (X) and temperature (Y) values of each data point are displayed above the graph. Determine the maximum temperature, t1, and the minimum temperature, t2 (round to the nearest 0.1°C).

Processing the data

1.	Use the equation t = t2 – t1 to determine t, the change in water temperature.

2.	Subtract to determine the volume of ice that was melted (V2 –V1).

3.	Find the mass of ice melted using the volume of melt (use 1.00 g/mL as the density of water).

4.	Use the equation given in the introduction of this experiment to calculate the energy (in joules) released by the 100 g of liquid water as it cooled through t.

5.	Now use the results obtained above to determine the heat of fusion, the energy required to melt one gram of ice (in J/g H2O).

6.	Use your answer to Step 5 and the molar mass of water to calculate the molar heat of fusion for ice (in kJ/mol H2O).

7.	Find the percent error for the molar heat of fusion value in Step 6. The accepted value for molar heat of fusion is 6.01 kJ/mol.

DATA and calculations

| Initial water temperature, t1 | °C |
| --- | --- |
| Final water temperature, t2 | °C |
| Change in water temperature, t | °C |
| Final water volume, V2 | mL |
| Initial water volume, V1 | mL |
| Volume of melt | mL |

Mass of ice melted g

Heat released by cooling water (q = Cp•m•Dt) J

J/g ice melted (heat of fusion) J/g

kJ/mol ice melted (molar heat of fusion) kJ/mol

Percent error %