Determining the Free Chlorine
Content of Swimming Pool Water

Physicians in the nineteenth century used chlorine water as a disinfectant. Upon the discovery that certain diseases were transmitted by water, it became common for municipalities to chlorinate public water supplies. It is now standard practice to add chlorine to swimming pools and hot tubs. Chlorine reacts with water to form hypochlorous acid (HOCl) and hypochlorite ion (ClO–).

Cl2 + H2O  HOCl + H+ + Cl–

HOCl  H+ + OCl–

The chlorine that exists in water as HOCl and OCl– is known as free chlorine. Free chlorine can kill bacteria, prevent algae growth, and oxidize iron to form a precipitate that can be removed from a pool by the filtering system. Swimming pool operators try to maintain a desired range of 1.0 to 1.5 mg/L of free chlorine for proper sanitation.

In this experiment, you will use a Colorimeter to determine the amount of free chlorine in a sample of swimming pool or hot tub water. You will first measure the absorbance of green light (565 nm) by aqueous solutions of known chlorine concentration. From the resulting graph of absorbance vs. free chlorine concentration (Beer’s law), you will be able to determine the free chlorine content of you swimming pool sample.

A compound called DPD (N, N-diethyl-p-phenylenediamine) is reacted with the chlorine in each sample. The DPD is oxidized, forming a magenta (red) product. The intensity of the sample’s color is directly proportional to its chlorine concentration.

Figure 2

MATERIALS

Figure 1

| TI-83 Plus or TI-84 Plus graphing calculator | free-chlorine standard (10 mg/L) |
| --- | --- |
| EasyData application | water samples from a pool or hot tub |
| data-collection interface | six DPD free-chlorine powder pillows |
| Vernier Colorimeter | 25 mL pipet |
| one cuvette | 10 mL pipet |
| tissues (preferably lint-free) | pipet pump or pipet bulb |
| six 50 mL beakers | stirring rod |
| 100 mL beaker | distilled water |

PROCEDURE

1.	Obtain and wear goggles.

2.	Label six clean, dry, 50 mL beakers 1–6. Transfer about 20 mL of the free-chlorine standard solution (10 mg/L) to a 100 mL beaker. CAUTION: Handle this solution with care. Avoid breathing its vapors. Use a 10 mL pipet to measure out exactly 1.00 mL of the free-chlorine standard solution into Beaker 1. Use a 25 mL pipet to add 24.00 mL of distilled water to Beaker 1. Continue using this procedure to prepare the free-chlorine solutions in Beakers 2–5, using the amounts of the free-chlorine standard solution and distilled water shown here:

| Beaker number | Free-chlorine standard (10 mg/L) (mL) | Distilled water (mL) | Free-chlorine concentration (mL) |
| --- | --- | --- | --- |
| 1 | 1.00 | 24.00 | 0.40 |
| 2 | 2.00 | 23.00 | 0.80 |
| 3 | 3.00 | 22.00 | 1.20 |
| 4 | 4.00 | 21.00 | 1.60 |
| 5 | 5.00 | 20.00 | 2.00 |

3.	Use a 25 mL pipet to transfer 25.00 mL of your swimming pool water sample to Beaker 6.

4.	Add one DPD free-chlorine powder pillow to each of the six labeled beakers. CAUTION: Handle the powder pillows with care. Mix the powder thoroughly into the sample using a stirring rod. Note: It is not important if a few small bits of powder do not dissolve.

5.	Turn on the calculator. Connect the Colorimeter, data-collection interface, and calculator.

6.	Prepare a blank by filling an empty cuvette 3/4 full with distilled water. Seal the cuvette with a lid. To correctly use a Colorimeter cuvette, remember:

- All cuvettes should be wiped clean and dry on the outside with a tissue.

- Handle cuvettes only by the top edge of the ribbed sides.

- All solutions should be free of bubbles.

- Always position the cuvette with its reference mark facing toward the white reference mark at the right of the cuvette slot on the Colorimeter.

7.	Set up EasyData for data collection and calibrate the Colorimeter.

- Start the EasyData application, if it is not already running.

- Select  from the Main screen, and then select New to reset the application.

- Select  from the Main screen, and then select Events with Entry.

- Place the blank in the cuvette slot of the Colorimeter and close the lid.

- Set the wavelength on the Colorimeter to 565 nm, then calibrate by pressing the CAL button on the Colorimeter.

8.	You are now ready to collect absorbance-concentration data for the five standard solutions.

- Select  from the Main screen.

- Empty the water from the cuvette. Using the solution in Beaker 1, rinse the cuvette twice with ~1 mL amounts and then fill it 3/4 full. Wipe the outside with a tissue, place it in the Colorimeter, and close the lid.

- When the value displayed on the calculator screen has stabilized, select . Enter 0.40 as the concentration in mg/L and select . The absorbance and concentration values have now been saved for the first solution.

- Discard the cuvette contents as directed by your instructor. Using the solution in Beaker 2, rinse the cuvette twice with ~1 mL amounts and then fill it 3/4 full. After closing the lid, wait for the value displayed on the calculator screen to stabilize and select . Enter 0.80 as the concentration in mg/L and select .

- Repeat the procedure for Beaker 3 (1.2 mg/L), Beaker 4 (1.6 mg/L), and Beaker 5 (2.0 mg/L). Note: Wait until Step 9 to measure the absorbance of the swimming pool water (Beaker 6).

- Select  to stop data collection. The absorbance and concentration values have now been saved for the standard solutions.

- Examine the data points along the curve on the displayed graph. As you move the cursor right or left, the concentration (X) and absorbance (Y) values of each data point are displayed above the graph. Record the absorbance values in your data table (round to the nearest 0.001).

- Select  to return to the Main screen.

9.	Determine the absorbance value of the swimming pool water. To do this:

- Obtain 10 mL of the swimming pool water to be tested.

- Rinse the cuvette twice with the swimming pool water and fill it about 3/4 full. Wipe the outside of the cuvette, place it into the Colorimeter, and close the lid.

- Monitor the absorbance value displayed on the calculator. When this value has stabilized, record the absorbance value for Trial 6, the swimming pool sample, in your data table (round to the nearest 0.001).

- Dispose of the remaining solution in the flask as directed by your instructor.

10.	Discard the solutions as directed by your instructor. Proceed directly to Steps 1–2 of Processing the Data.

Processing the data

1.	To determine the concentration of the swimming pool water sample, you can use your calculator and interpolate along the regression line on your Beer’s law curve. Use the following method to convert the absorbance value of the unknown to concentration, in mg/L.

- Select  from the Main screen.

- Select , then Linear Fit. The linear-regression statistics for these two lists are displayed for the equation in the form

y = ax + b

- where y is absorbance, x is concentration, a is the slope, and b is the y-intercept. Note: One indicator of the quality of your data is the size of b. It is a very small value if the regression line passes through or near the origin. The correlation coefficient, r, indicates how closely the data points match up with (or fit) the regression line. A value of 1.00 indicates a nearly perfect fit.

- To display the linear-regression curve on the graph of absorbance vs. concentration, select . This graph should indicate a direct relationship between absorbance and concentration, a relationship known as Beer’s law. The regression line should closely fit the five data points and pass through (or near) the origin of the graph.

- To interpolate along the curve, press . A cursor is displayed on the regression line, along with its X and Y coordinates above the graph. Use  or  to move the cursor to an absorbance value (Y value) that is closest to the absorbance reading you obtained in 
Step 9. The free chlorine concentration of your swimming pool sample, in mg/L, will be equal to the corresponding X value. Record this value.

2.	Print a graph of absorbance vs. concentration, with a regression line and interpolated unknown concentration displayed.

DATA and calculations

| Trial | Concentration (mg/L) | Absorbance |
| --- | --- | --- |
| 1 | 0.40 |  |
| 2 | 0.80 |  |
| 3 | 1.20 |  |
| 4 | 1.60 |  |
| 5 | 2.00 |  |
| 6 | Swimming pool sample |  |

| Concentration of swimming pool sample | mg/L |
| --- | --- |