Chemical Equilibrium:
Finding a Constant, Kc

The purpose of this lab is to experimentally determine the equilibrium constant, Kc, for the following chemical reaction:

Fe3+(aq) + SCN–(aq)  FeSCN2+(aq)

iron(III)    thiocyanate        thiocyanoiron(III)

When Fe3+ and SCN– are combined, equilibrium is established between these two ions and the FeSCN2+ ion. In order to calculate Kc for the reaction, it is necessary to know the concentrations of all ions at equilibrium: [FeSCN2+]eq, [SCN–]eq, and [Fe3+]eq. You will prepare four equilibrium systems containing different concentrations of these three ions. The equilibrium concentrations of the three ions will then be experimentally determined. These values will be substituted into the equilibrium constant expression to see if Kc is indeed constant.

In order to determine [FeSCN2+]eq, you will use the Colorimeter shown in Figure 1. The FeSCN2+ ion produces solutions with a red color. Because the red solutions absorb blue light very well, the blue LED setting on the Colorimeter is used. The Colorimeter measures the amount of blue light absorbed by the colored solutions (absorbance, A). By comparing the absorbance of each equilibrium system, Aeq, to the absorbance of a standard solution, Astd, you can determine [FeSCN2+]eq. The standard solution has a known FeSCN2+ concentration.

Figure 1

To prepare the standard solution, a very large concentration of Fe3+ will be added to a small initial concentration of SCN– (hereafter referred to as [SCN–]i. The [Fe3+] in the standard solution is 100 times larger than [Fe3+] in the equilibrium mixtures. According to LeChatelier’s principle, this high concentration forces the reaction far to the right, using up nearly 100% of the SCN– ions. According to the balanced equation, for every one mole of SCN– reacted, one mole of FeSCN2+ is produced. Thus [FeSCN2+]std is assumed to be equal to [SCN–]i.

Assuming [FeSCN2+] and absorbance are related directly (Beer’s law), the concentration of FeSCN2+ for any of the equilibrium systems can be found by:

[FeSCN2+]eq =  X [FeSCN2+]std

Knowing the [FeSCN2+]eq allows you to determine the concentrations of the other two ions at equilibrium. For each mole of FeSCN2+ ions produced, one less mole of Fe3+ ions will be found in the solution (see the 1:1 ratio of coefficients in the equation on the previous page). The [Fe3+] can be determined by:

[Fe3+]eq = [Fe3+]i – [FeSCN2+]eq

Because one mole of SCN- is used up for each mole of FeSCN2+ ions produced, [SCN-]eq can be determined by:

[SCN–]eq = [SCN–]i – [FeSCN2+]eq

Knowing the values of [Fe3+]eq, [SCN–]eq, and [FeSCN2+]eq, you can now calculate the value of Kc, the equilibrium constant.

OBJECTIVE

In this experiment, you will determine the equilibrium constant, Kc, for the following chemical reaction:

Fe3+(aq) + SCN–(aq)    FeSCN2+(aq)

iron(III)     thiocyanate	thiocyanoiron(III)

MATERIALS

| LabQuest | 0.0020 M KSCN |
| --- | --- |
| LabQuest App | 0.0020 M Fe(NO3)3 (in 1.0 M HNO3) |
| Vernier Colorimeter | 0.200 M Fe(NO3)3 (in 1.0 M HNO3) |
| one cuvette | four pipets |
| three 100 mL beakers | pipet bulb or pipet pump |
| five 20 X 150 mm test tubes | tissues (preferably lint-free) |
| thermometer |  |

PROCEDURE

1.	Obtain and wear goggles.

2.	Label four 20 X 150 mm test tubes 1–4. Pour about 30 mL of 0.0020 M Fe(NO3)3 into a clean, dry 100 mL beaker. Pipet 5.0 mL of this solution into each of the four labeled test tubes. Use a pipet pump or bulb to pipet all solutions. CAUTION: Fe(NO3)3 solutions in this experiment are prepared in 1.0 M HNO3 and should be handled with care. Pour about 25 mL of the 0.0020 M KSCN into another clean, dry 100 mL beaker. Pipet 2, 3, 4 and 5 mL of this solution into Test Tubes 1–4, respectively. Obtain about 25 mL of distilled water in a 100 mL beaker. Then pipet 3, 2, 1 and 0 mL of distilled water into Test Tubes 1–4, respectively, to bring the total volume of each test tube to 10 mL. Mix each solution thoroughly with a stirring rod. Be sure to clean and dry the stirring rod after each mixing. Measure and record the temperature of one of the above solutions to use as the temperature for the equilibrium constant, Kc. Volumes added to each test tube are summarized below:

| Test Tube Number | Fe(NO3)3 (mL) | KSCN (mL) | H2O (mL) |
| --- | --- | --- | --- |
| 1 | 5 | 2 | 3 |
| 2 | 5 | 3 | 2 |
| 3 | 5 | 4 | 1 |
| 4 | 5 | 5 | 0 |

3.	Prepare a standard solution of FeSCN2+ by pipetting 18 mL of 0.200 M Fe(NO3)3 into a 20 X 150 mm test tube labeled “5”. Pipet 2 mL of 0.0020 M KSCN into the same test tube. Stir thoroughly.

4.	Connect the Colorimeter to LabQuest and choose New from the File menu.

5.	Prepare a blank by filling an empty cuvette 3/4 full with distilled water. Seal the cuvette with a lid. To correctly use a Colorimeter cuvette, remember:

- All cuvettes should be wiped clean and dry on the outside with a tissue.

- Handle cuvettes only by the top edge of the ribbed sides.

- All solutions should be free of bubbles.

- Always position the cuvette with its reference mark facing toward the white reference mark at the top of the cuvette slot on the Colorimeter.

6.	Calibrate the Colorimeter.

- Place the blank in the cuvette slot of the Colorimeter and close the lid.

- Press the < or > button on the Colorimeter to set the wavelength to 470 nm (Blue). Then calibrate by pressing the CAL button on the Colorimeter. When the LED stops flashing, the calibration is complete.

7.	On the Meter screen, tap Mode. Change the data-collection mode to Selected Events. Select OK.

8.	You are now ready to collect absorbance data for the four equilibrium systems and the standard solution.

- Start data collection.

- Empty the water from the cuvette. Using the solution in Test Tube 1, rinse the cuvette twice with ~1 mL amounts and then fill it 3/4 full. Wipe the outside with a tissue, place it in the Colorimeter, and close the lid.

- When the value displayed on the screen has stabilized, tap Keep to store the absorbance value for the first trial.

- Discard the cuvette contents as directed by your instructor. Using the solution in Test Tube 2, rinse the cuvette twice with ~1 mL amounts, and then fill it 3/4 full. After closing the lid, wait for the value displayed on the screen to stabilize. Tap Keep to store the reading for the second trial.

- Repeat the Step-d procedure to find the absorbance of the solutions in Test Tubes 3, 4, and 5 (the standard solution).

- Stop data collection.

- Examine the data points along the curve on the displayed graph. To examine the data pairs on the graph, tap any data point. Record the displayed absorbance values corresponding to each point in your data table.

PROCESSING THE DATA

1.	Write the Kc expression for the reaction in the Data and Calculation table.

2.	Calculate the initial concentration of Fe3+, based on the dilution that results from adding KSCN solution and water to the original 0.0020 M Fe(NO3)3 solution. See Step 2 of the procedure for the volume of each substance used in Trials 1–4. Calculate [Fe3+]i using the equation:

[Fe3+]i =  X (0.0020 M)

This should be the same for all four test tubes.

3.	Calculate the initial concentration of SCN–, based on its dilution by Fe(NO3)3 and water:

[SCN–]i = X (0.0020 M)

In Test Tube 1, [SCN–]i = (2 mL / 10 mL)(.0020 M) = .00040 M. Calculate this for the other three test tubes.

4.	[FeSCN2+]eq is calculated using the formula:

[FeSCN2+]eq =  X [FeSCN2+]std

where Aeq and Astd are the absorbance values for the equilibrium and standard test tubes, respectively, and [FeSCN2+]std = (1/10)(0.0020) = 0.00020 M. Calculate [FeSCN2+]eq for each of the four trials.

5.	[Fe3+]eq: Calculate the concentration of Fe3+ at equilibrium for Trials 1-4 using the equation:

[Fe3+]eq = [Fe3+]i – [FeSCN2+]eq

6.	[SCN–]eq: Calculate the concentration of SCN- at equilibrium for Trials 1-4 using the equation:

[SCN–]eq = [SCN–]i – [FeSCN2+]eq

7.	Calculate Kc for Trials 1–4. Be sure to show the Kc expression and the values substituted in for each of these calculations.

8.	Using your four calculated Kc values, determine an average value for Kc. How constant were your Kc values?

DATA AND CALCULATIONS

| Absorbance | Trial 1 _______ |  | Trial 2 _______ |  | Trial 3 _______ |  | Trial 4 _______ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Absorbance of standard (Trial 5) |  | _______ |  | Temperature |  | _______ °C |  |

| Kc expression Kc = |  |  |  |  |
| --- | --- | --- | --- | --- |
| [Fe3+]i |  |  |  |  |
| [SCN–]i |  |  |  |  |
| [FeSCN2+]eq |  |  |  |  |
| [Fe3+]eq |  |  |  |  |
| [SCN–]eq |  |  |  |  |
| Kc value |  |  |  |  |
| Average of Kc values Kc = ________ at ________°C |  |  |  |  |