Determining the Free Chlorine
Content of Swimming Pool Water

Physicians in the nineteenth century used chlorine water as a disinfectant. Upon the discovery that certain diseases were transmitted by water, it became common for municipalities to chlorinate public water supplies. It is now standard practice to add chlorine to swimming pools and hot tubs. Chlorine reacts with water to form hypochlorous acid (HOCl) and hypochlorite ion (ClO–).

Cl2 + H2O  HOCl + H+ + Cl–

HOCl  H+ + OCl–

The chlorine that exists in water as HOCl and OCl– is known as free chlorine. Free chlorine can kill bacteria, prevent algae growth, and oxidize iron to form a precipitate that can be removed from a pool by the filtering system. Swimming pool operators try to maintain a desired range of 1.0 to 1.5 mg/L of free chlorine for proper sanitation.

Figure 1

In this experiment, you will use a Colorimeter to determine the amount of free chlorine in a sample of swimming pool or hot tub water. You will first measure the absorbance of green light (565 nm) by aqueous solutions of known chlorine concentration. From the resulting graph of absorbance vs. free chlorine concentration (Beer’s law), you will be able to determine the free chlorine content of your swimming pool sample.

A compound called DPD (N, N-diethyl-p-phenylenediamine) is reacted with the chlorine in each sample. The DPD is oxidized, forming a magenta (red) product. The intensity of the sample’s color is directly proportional to its chlorine concentration.

OBJECTIVES

In this experiment, you will

- Prepare free chlorine standard solutions.

- Use a Colorimeter to measure the absorbance of each standard solution.

- Plot a graph of absorbance of free chlorine vs. concentration.

- Use a Colorimeter and your Beer’s law plot to determine the amount of free chlorine in a sample of swimming pool or hot tub water.

MATERIALS

| LabQuest | water samples from a pool or hot tub |
| --- | --- |
| LabQuest App | six DPD free-chlorine powder pillows |
| Vernier Colorimeter | 25 mL pipet |
| one cuvette | 10 mL pipet |
| tissues (preferably lint-free) | pipet pump or pipet bulb |
| six 50 mL beakers | stirring rod |
| 100 mL beaker | distilled water |
| free-chlorine standard (10 mg/L) |  |

PROCEDURE

1.	Obtain and wear goggles.

2.	Label six clean, dry, 50 mL beakers 1–6. Transfer about 20 mL of the free-chlorine standard solution (10 mg/L) to a 100 mL beaker. CAUTION: Handle this solution with care. Avoid breathing its vapors. Use a 10 mL pipet to measure out exactly 1.00 mL of the free-chlorine standard solution into Beaker 1. Use a 25 mL pipet to add 24.00 mL of distilled water to Beaker 1. Continue using this procedure to prepare the free-chlorine solutions in Beakers 2–5, using the amounts of the free-chlorine standard solution and distilled water shown here:

| Beaker number | Free-chlorine standard (10 mg/L) (mL) | Distilled water (mL) | Free-chlorine concentration (mL) |
| --- | --- | --- | --- |
| 1 | 1.00 | 24.00 | 0.40 |
| 2 | 2.00 | 23.00 | 0.80 |
| 3 | 3.00 | 22.00 | 1.20 |
| 4 | 4.00 | 21.00 | 1.60 |
| 5 | 5.00 | 20.00 | 2.00 |

3.	Use a 25 mL pipet to transfer 25.00 mL of your swimming pool water sample to Beaker 6.

4.	Add one DPD free-chlorine powder pillow to each of the six labeled beakers. CAUTION: Handle the powder pillows with care. Mix the powder thoroughly into the sample using a stirring rod. Note: It is not important if a few small bits of powder do not dissolve.

5.	Connect the Colorimeter to LabQuest and choose New from the File menu.

6.	Prepare a blank by filling an empty cuvette 3/4 full with distilled water. Seal the cuvette with a lid. To correctly use a Colorimeter cuvette, remember:

- All cuvettes should be wiped clean and dry on the outside with a tissue.

- Handle cuvettes only by the top edge of the ribbed sides.

- All solutions should be free of bubbles.

- Always position the cuvette with its reference mark facing toward the white reference mark at the top of the cuvette slot on the Colorimeter.

7.	Calibrate the Colorimeter.

- Place the blank in the cuvette slot of the Colorimeter and close the lid.

- Press the < or > button on the Colorimeter to set the wavelength to 565 nm (Green). Then calibrate by pressing the CAL button on the Colorimeter. When the LED stops flashing, the calibration is complete.

8.	Set up the data-collection mode.

- On the Meter screen, tap Mode. Change the data-collection mode to Events with Entry.

- Enter the Name (Concentration) and Units (mol/L). Select OK.

9.	You are now ready to collect absorbance-concentration data for the five standard solutions.

- Start data collection.

- Empty the water from the cuvette. Using the solution in Beaker 1, rinse the cuvette twice with ~1-mL amounts and then fill it 3/4 full. Wipe the outside with a tissue, place it in the Colorimeter, and close the lid.

- When the value displayed on LabQuest has stabilized, tap Keep and enter 0.40 as the concentration in mg/L. Select OK. The absorbance and concentration values have now been saved for the first solution.

- Discard the cuvette contents as directed by your instructor. Using the solution in Beaker 2, rinse the cuvette twice with ~1 mL amounts and then fill it 3/4 full. After closing the lid, wait for the value to stabilize and tap Keep. Enter 0.80 as the concentration in mg/L, and select OK.

- Repeat the procedure for Beaker 3 (1.2 mg/L), Beaker 4 (1.6 mg/L), and Beaker 5 (2.0 mg/L). Note: Wait until Step 10 to measure the absorbance of the swimming pool water (Beaker 6).

- Stop data collection.

- Examine the data pairs on the absorbance vs. concentration graph. As you tap each data point, the absorbance and concentration values are displayed to the right of the graph. Record the absorbance and concentration data values in your data table.

10.	Determine the absorbance value of the swimming pool water.

- Obtain 10 mL of the swimming pool water to be tested.

- Rinse the cuvette twice with the swimming pool water and fill it about 3/4 full. Wipe the outside of the cuvette, place it into the Colorimeter, and close the lid.

- Tap the Meter tab to return to the Meter screen and monitor the absorbance value. When this value has stabilized, record the absorbance value for Trial 6, the swimming pool sample, in your data table.

11.	Discard the solutions as directed by your instructor. Proceed directly to Steps 1–2 of Processing the Data.

Processing the data

1.	To determine the concentration of the swimming pool water sample, you can interpolate along the regression line on your Beer’s law curve. Use the following method to convert the absorbance value of the unknown to concentration, in mg/L.

- Tap the Graph tab and choose Curve Fit from the Analyze menu.

- Select Linear as the Fit Equation. The linear-regression statistics for these two data columns are displayed for the equation in the form

y = mx + b

- where x is concentration, y is absorbance, m is the slope, and b is the y-intercept. Note: One indicator of the quality of your data is the size of b. It is a very small value if the regression line passes through or near the origin. The correlation coefficient, r, indicates how closely the data points match up with (or fit) the regression line. A value of 1.00 indicates a nearly perfect fit.

- Select OK. The graph should indicate a direct relationship between absorbance and concentration, a relationship known as Beer’s law. The regression line should closely fit the five data points and pass through (or near) the origin of the graph.

- Choose Interpolate from the Analyze menu. Interpolate along the regression curve to determine the concentration of the unknown solution. Tap any point on the regression curve (or use the ► or ◄ key of LabQuest) to advance to the absorbance value that is closest to the absorbance reading you obtained in Step 10. The corresponding free chlorine concentration of your swimming pool sample, in mg/L, will be displayed to the right of the graph. Record this value in your data table.

2.	(optional) Print a graph of absorbance vs. concentration, with a regression line and interpolated unknown concentration displayed.

DATA and calculations

| Trial | Concentration (mg/L) | Absorbance |
| --- | --- | --- |
| 1 | 0.40 |  |
| 2 | 0.80 |  |
| 3 | 1.20 |  |
| 4 | 1.60 |  |
| 5 | 2.00 |  |
| 6 | Swimming pool sample |  |

| Concentration of swimming pool sample | mg/L |
| --- | --- |