Determining the Phosphoric Acid
Content in Soft Drinks

Phosphoric acid is one of several weak acids that exist in carbonated beverages. It is a component of all cola soft drinks. Phosphoric acid has a much higher concentration than other acids in a container of soft drink, so its concentration can be determined by a simple acid-base titration.

In this experiment, you will titrate a sample of a cola soft drink with sodium hydroxide solution and determine the concentration of phosphoric acid, H3PO4. Hydrogen ions from the first dissociation of phosphoric acid react with hydroxide ions from the NaOH in a one-to-one ratio in the overall reaction:

H3PO4(aq) + OH–(aq)  H2O(l) + H2PO4–(aq)

In this experiment, you will use a pH Sensor to monitor pH as you titrate. The region of most rapid pH change will then be used to determine the equivalence point. The volume of NaOH titrant used at the equivalence point will be used to determine the molarity of the H3PO4.

OBJECTIVES

In this experiment, you will

- Use a pH Sensor to monitor pH during the titration of phosphoric acid in a cola soft drink.

- Using the titration equivalence point, determine the molarity of H3PO4.

MATERIALS

| LabQuest | various cola soft drinks, decarbonated |
| --- | --- |
| LabQuest App | distilled water |
| Vernier pH Sensor | ring stand |
| 50 mL buret | utility clamp |
| 100 mL graduated cylinder | magnetic stirrer (if available) |
| 250 mL beaker | stirring bar |
| 0.050 M NaOH |  |

PROCEDURE

1.	Obtain and wear goggles.

2.	Use a graduated cylinder to measure out 40 mL of a decarbonated cola beverage and 60 mL of distilled water into a 250 mL beaker. CAUTION: Do not eat or drink in the laboratory.

3.	Place the beaker on a magnetic stirrer and add a stirring bar. If no magnetic stirrer is available, you need to stir with a stirring rod during the titration.

4.	Connect the pH Sensor to LabQuest and choose New from the File menu. If you have an older sensor that does not auto-ID, manually set up the sensor.

5.	Use a utility clamp to suspend a pH Sensor on a ring stand as shown in Figure 1. Position the pH Sensor in the HCl solution and adjust its position so it is not struck by the stirring bar.

6.	Obtain a 50 mL buret and rinse the buret with a few mL of the 0.050 M NaOH solution. CAUTION: Sodium hydroxide solution is caustic. Avoid spilling it on your skin or clothing. Dispose of the rinse solution as directed by your teacher. Use a utility clamp to attach the buret to the ring stand as shown in Figure 1. Fill the buret a little above the 0.00 mL level of the buret with 0.050 M NaOH solution. Drain a small amount of NaOH solution so it fills the buret tip and leaves the NaOH at the 0.00 mL level of the buret. Record the precise concentration of the NaOH solution in your data table.

Figure 1

7.	Set up the data-collection mode.

- On the Meter screen, tap Mode. Change the data-collection mode to Events with Entry.

- Enter the Name (Volume) and Units (mL). Select OK.

8.	You are now ready to perform the titration. This process goes faster if one person manipulates and reads the buret while another person enters volumes.

- Start data collection.

- Before you have added any NaOH solution, tap Keep and enter 0 as the buret volume in mL. Select OK to store the first data pair for this experiment.

- Add 0.5 mL of NaOH solution. When the pH stabilizes, tap Keep and enter the current buret reading. Select OK.

- Continue to add 0.5 mL increments, entering the buret level after each increment. When the pH has leveled off (near pH 10), stop data collection.

9.	Examine the data on the graph of pH vs. volume to find the equivalence point—that is, the 0.5 mL volume increment that resulted in the largest increase in pH. As you tap each data point (or use the ► or ◄ keys on LabQuest), the pH and volume values are displayed. Go to the region of the graph with the large increase in pH. Find the NaOH volume (in mL) just before this jump. Record this value in the data table. Then record the NaOH volume after the 0.5 mL addition producing the largest pH increase.

10.	Print a copy of the graph of pH vs. volume. Then print a copy of the NaOH volume data and the pH data for the titration.

11.	Dispose of the beaker contents as directed by your teacher. Rinse the pH Sensor and return it to the storage solution.

Processing the data

1.	Use your printed graph and data table to confirm the volume of NaOH titrant you recorded before and after the largest increase in pH values upon the addition of 0.5 mL of NaOH solution.

2.	Determine the volume of NaOH added at the first equivalence point. To do this, add the two NaOH values determined above and divide by two.

3.	Calculate the number of moles of NaOH used.

4.	See the equation for the neutralization reaction given in the introduction. Determine the number of moles of H3PO4 reacted.

5.	Recall that you pipeted out 40.0 mL of the beverage for the titration. Calculate the H3PO4 concentration.

DATA TABLE

| Concentration of NaOH | M |
| --- | --- |
| NaOH volume added before the largest pH increase | mL |
| NaOH volume added after the largest pH increase | mL |

| Volume of NaOH added at equivalence point | mL |
| --- | --- |
| Moles NaOH | mol |
| Moles H3PO4 | mol |
| Concentration of H3PO4 | mol/L |