A Hot Hand

In this class, you will often work in a team using a calculator to do science. You will measure the temperature of the palm of your hand and the palm temperatures of your teammates in this experiment. In the process, you will learn how to use DataMate, a calculator program you will be using throughout the school year, and how to use Temperature Probes. You will also get to know your teammates better.

OBJECTIVES

In this experiment, you will

- Use a Temperature Probe to measure temperature.

- Calculate temperature averages.

- Compare results.

MATERIALS

| LabPro or CBL 2 interface | beaker |
| --- | --- |
| TI graphing calculator | water |
| DataMate program | paper towel |
| Temperature Probe |  |

Figure 1

PROCEDURE

1.	Plug the Temperature Probe into Channel 1 of the LabPro or CBL 2 interface. Use the link cable to connect the TI graphing calculator to the interface. Firmly press in the cable ends.

2.	Turn on the calculator and follow these steps to start the DATAMATE program.

TI-73 and TI-83 Calculators:

Press , then press the calculator key for the number that precedes the DATAMATE program (usually ). Press . You are now at the main screen of the program. Press  to reset the program.

TI-83 Plus Calculators:

Press , then press the calculator key for the number that precedes the DATAMATE program. You are now at the main screen of the program. Press  to reset the program.

TI-86 Calculators:

Press , press  to select <NAMES>, and press a menu key to select <DATAM> (usually ). Press . You are now at the main screen of the program. Press  to reset the program.

TI-89, TI-92, and TI-92 Plus Calculators:

Press  [VAR-LINK]. Use  or the cursor pad to scroll down to “datamate”, then press . Press  to complete the open parenthesis that follows “datamate” on the entry line and press . You are now at the main screen of the program. Press  to reset the program.

3.	Set up the calculator and interface for the correct Temperature Probe.

- Select SETUP from the main screen.

- If the calculator displays a Temperature Probe in CH 1, proceed directly to Step 4. If it does not, continue with this step to set up your sensor manually.

- Press  to select CH 1.

- Select TEMPERATURE from the SELECT SENSOR menu.

- Select the correct Temperature Probe (in °C) from the TEMPERATURE menu.

4.	Set up the calculator and interface for data collection.

- Use  and  to select MODE and press .

- Select TIME GRAPH from the SELECT MODE menu.

- Select CHANGE TIME SETTINGS from the TIME GRAPH SETTINGS menu.

- Enter 1 as the time between samples in seconds.

- Enter 60 as the number of samples. Data collection will last 60 seconds.

- Select OK to return to the setup screen.

- Select OK again to return to the main screen.

5.	Measure the temperature of the palm of your hand.

- Select START to begin data collection.

- Pick up the Temperature Probe and hold its tip in the palm of your hand as shown in Figure 1. Data collection will end when 60 seconds have gone by.

6.	Record your highest temperature.

- When data collection is complete after 60 seconds, a graph of temperature vs. time will be displayed. Use  to examine data points along the curve. As you move the cursor right or left, the time (X) and temperature (Y) values of each data point are displayed below the graph.

- Record your highest temperature (to the nearest 0.1°C).

- Press  to return to the main screen.

7.	Prepare the Temperature Probe for the next run.

- Cool the Temperature Probe by placing it into a beaker of room-temperature water until its temperature reaches the temperature of the water. The temperature of the probe is displayed at the top of the main screen.

- Use a paper towel to dry the probe. Be careful not to warm the probe as you dry it.

8.	Repeat Steps 5–7 for each person in your team.

DATA

| Student name | Highest temperature |
| --- | --- |
|  | °C |
|  | °C |
|  | °C |
|  | °C |
| Team average | °C |

PROCESSING THE DATA

1.	Calculate your team average for the highest temperatures. Record the result in the data table above.

- How did the highest temperatures of your teammates compare?

3.	Who had the “hottest hand”?

EXTENSION

1.	Determine the class average for highest temperature.