Reflectivity of Light

Light is reflected differently from various surfaces and colors. In this experiment, you will be measuring the percent reflectivity of various colors. You will measure reflection values from paper of the various colors using a Light Sensor and then compare these values to the reflection value of aluminum foil. You will then calculate percent reflectivity using the relationship:

% Reflectivity  =  X 100

OBJECTIVES

In this experiment, you will

- Use a Light Sensor to measure reflected light.

- Calculate percent reflectivity of various colors.

- Make conclusions using the results of the experiment.

MATERIALS

| LabPro or CBL 2 interface | white paper |
| --- | --- |
| TI graphing calculator | black paper |
| DataMate program | 2 other pieces of colored paper |
| Light Sensor | ring stand |
| aluminum foil | utility clamp |

Figure 1

PROCEDURE

1.	Use a utility clamp and ring stand to fasten a Light Sensor 5 cm above a piece of colored paper as shown in Figure 1. The classroom lights should be on. Note: If you are using a Vernier Light Sensor, set it to the 0–6000 lux position.

2.	Plug the Light Sensor into Channel 1 of the LabPro or CBL 2 interface. Use the link cable to connect the TI graphing calculator to the interface. Firmly press in the cable ends.

3.	Turn on the calculator and start the DATAMATE program. Press  to reset the program.

4.	Set up the calculator and interface for the Light Sensor.

- If the calculator displays a Light Sensor in CH 1, proceed directly to Step 5. If it does not, continue with this step to set up your sensor manually.

- Select SETUP from the main screen.

- Press  to select CH 1.

- Select LIGHT from the SELECT SENSOR menu.

- Select LIGHT 6000 (LX) from the menu.

- Select OK to return to the main screen. The light reading is now displayed at the top of the main screen.

5.	When the reading stabilizes, record the color and the reflected light value.

6.	Make and record readings for aluminum, black, white, and two other colors.

DATA

| Color | Aluminum | Black | White | __________ | __________ |
| --- | --- | --- | --- | --- | --- |
| Reflection value |  |  |  |  |  |
| Percent reflectivity | 100% | % | % | % | % |

PROCESSING THE DATA

1.	Calculate the percent reflectivity of each color using the formula given in the introduction. Show your work in the data table above.

2.	Which color, other than aluminum, has the highest reflectivity?

3.	Which color has the lowest reflectivity?

4.	What surfaces might give a planet a high reflectivity? Explain.

5.	Does the planet Earth have high reflectivity? Why or why not?

EXTENSION

1.	Design an experiment to test the reflectivity of sand, soil, water, and other materials. Perform the experiment you designed.