Solar Homes

Alternative energy sources are energy sources other than the nonrenewable fossil fuels—coal, petroleum, and natural gas. Solar energy, or energy from the sun, is one energy source alternative. A passive solar heating system uses no pumps, fans, or mechanical devices. Insulation and heat storage are important factors in such a system. A thermal mass is a material that absorbs and stores heat. Thermal masses can keep a home from heating or cooling too fast. In this experiment, you will examine the effectiveness of a thermal mass. You will then use what you learn to design and build a model solar home.

OBJECTIVES

In this experiment, you will

- Measure temperature.

- Find the relationship between thermal mass and the ability of a solar home to retain heat.

- Design, build, and test a model solar home.

MATERIALS

| LabPro or CBL 2 interface | empty bottle with screw-on cap |
| --- | --- |
| TI graphing calculator | room-temperature water |
| DataMate program | lamp with 100 watt bulb |
| 2 Temperature Probes | 2 pieces of cardboard to cover the model |
| 2 model solar homes | solar home windows |
| masking tape | clock |

Figure 1

PROCEDURE

Part I

1.	Obtain two model solar homes and position them 20 cm apart. The window sides should face each other as shown in Figure 1.

2.	Fill a bottle with room temperature water. This will be the thermal mass for Part I. Firmly tighten the bottle cap and lay the filled bottle inside one of the model solar homes. Leave the other model home empty. Tape both model homes shut.

3.	Position a lamp to shine down between the model solar homes as shown in Figure 1. The lamp bulb should be 10 cm above the tabletop. It should be the same distance from each of the model homes. Do not turn on the lamp until instructed to do so in Step 10.

4.	Position Temperature Probe 1 in the model solar home with no thermal mass and Temperature Probe 2 in the model solar home with the thermal mass. In both cases, pass half of the probe through the hole provided. Make sure the probe is not in direct light from the lamp.

5.	Plug Temperature Probe 1 into Channel 1 of the LabPro or CBL 2 interface. Plug Temperature Probe 2 into Channel 2. Use the link cable to connect the TI graphing calculator to the interface. Firmly press in the cable ends.

6.	Turn on the calculator and start the DATAMATE program. Press  to reset the program.

7.	Set up the calculator and interface for the two Temperature Probes.

- Select SETUP from the main screen.

- If the calculator displays the correct Temperature Probes in CH 1 and CH 2, proceed directly to Step 8. If it does not, continue with this step to set up your sensors manually.

- Press  to select CH 1.

- Select TEMPERATURE from the SELECT SENSOR menu.

- Select the correct Temperature Probe (in °C) from the TEMPERATURE menu.

- Use  to select CH 2, then press .

- Select TEMPERATURE from the SELECT SENSOR menu.

- Select the correct Temperature Probe (in °C) from the TEMPERATURE menu.

8.	Set up the calculator and interface for data collection.

- Use  and  to select MODE and press .

- Select TIME GRAPH from the SELECT MODE menu.

- Select CHANGE TIME SETTINGS from the TIME GRAPH SETTINGS menu.

- Enter 240 as the time between samples in seconds.

- Enter 20 as the number of samples. Data collection will last 4800 seconds (80 minutes).

- Select OK to return to the setup screen.

- Select OK again to return to the main screen.

9.	You will be collecting data for 80 continuous minutes. During the first 40 minutes, the light will be on. After 40 minutes have passed, the light will be turned off and the windows of the model solar homes will be covered. Data will be collected for 40 more minutes with the light off.

10.	Select START to begin data collection. After you hear a beep, switch on the light bulb.

11.	After 40 minutes, turn the light off and cover the window of each model solar home with a piece of cardboard. Data collection will end when 4800 seconds (80 minutes) have passed.

12.	Determine the maximum temperature reached in each of the model solar homes.

- After data collection is complete, a graph of temperature vs. time will be displayed. Use  to examine data points along the curve for Probe 1. As you move the cursor right or left, the time (X) and temperature (Y) values of each data point are displayed below the graph. Note: P1 appears in the upper right corner of the screen.

- Move the cursor to the highest point on Curve P1.

- Record the Probe 1 (no thermal mass) maximum temperature (to the nearest 0.1°C).

- Use  to move the cursor to the curve for Probe 2. Note: P2 now appears in the upper right corner of the screen. Use  to examine data points along the curve for Probe 2.

- Record the Probe 2 (with thermal mass) maximum temperature (to the nearest 0.1°C).

13.	Using the Step 12 procedure, determine and record the temperature at 80 minutes (4800 seconds) for each probe.

14.	Sketch or print the graph as directed by your teacher.

DATA

|  | Probe 1 No thermal mass | Probe 2 With thermal mass |
| --- | --- | --- |
| Maximum temperature | °C | °C |
| Temperature at 80 minutes | °C | °C |
| Temperature change | °C | °C |

PROCESSING THE DATA

1.	In the space provided in the data table, subtract to find the temperature changes. Which model solar home cooled more?

2.	How are the “no thermal mass” and the “with thermal mass” curves similar? How are they different?

3.	Which model solar home heated more slowly?

4.	Which model solar home cooled more slowly?

5.	What effect does the thermal mass have on the heating and cooling of a model solar home?

6.	What are some advantages of heating a home with solar energy?

7.	What is a disadvantage of heating a home with solar energy?

Part II  Design Problem

1.	Using the information you gained in this and other experiments, design and build a model solar home that cools more slowly than the one you tested in Part I.

- Begin with a model solar home like the one used in Part I.

- You may add no more than 3 cm to the thickness of the walls.

- You may use no more than 600 mL of thermal mass.

- Your home must have a window with an area of at least 150 cm2.

- When testing your model, first use a lamp to raise the inside temperature to equal the maximum temperature inside the “with thermal mass” model in Part I. Then turn off the lamp, cover the window, and monitor temperature for 40 minutes as in Part I.

- Compare the results with your Part I results and the Part II results of your classmates.

- Explain why you chose the materials you did.

EXTENSIONS

1.	Run the experiment for two or more consecutive “daily” cycles of four hours or longer.

2.	Design an experiment to test other types of thermal mass, such as stones or phase-change materials.

3.	Design an experiment to test other variables affecting a solar home, such as color, window material, window size, and insulation type.