A Good Cold Pack

Cold packs are used to treat sprained ankles and similar injuries. A cold pack is typically made of a thin plastic inner bag containing water. That bag, in turn, is surrounded by a heavier plastic bag containing a solid substance. When the pack is twisted, the inner bag breaks and releases the water. As the solid substance dissolves in the water, energy is absorbed and the resulting mixture gets colder.

In this experiment, you will first determine temperature changes as several different solid substances dissolve in water. You will then develop and test a plan for making the best cold pack using 3.0 grams of one of the substances and the best amount of water.

OBJECTIVES

In this experiment, you will

- Measure temperature.

- Determine temperature changes as solid substances dissolve in water.

- Design and test a plan for making the best cold pack.

- Report your results.

MATERIALS

| LabPro or CBL 2 interface | 10 mL graduated cylinder |
| --- | --- |
| TI graphing calculator | water |
| DataMate program | ammonium chloride, NH4Cl |
| Temperature Probe | citric acid, H3C6H5O7 |
| balance | potassium chloride, KCl |
| weighing paper | sodium bicarbonate, NaHCO3 |
| 50 mL beaker | sodium carbonate, Na2CO3 |
| 250 mL beaker |  |

Figure 1

PROCEDURE

Part I  Finding Temperature Changes

1.	Obtain and wear goggles.

2.	Plug the Temperature Probe into Channel 1 of the LabPro or CBL 2 interface. Use the link cable to connect the TI graphing calculator to the interface. Firmly press in the cable ends.

3.	Turn on the calculator and start the DATAMATE program. Press  to reset the program.

4.	Set up the calculator and interface for the Temperature Probe.

- Select SETUP from the main screen.

- If the calculator displays the correct Temperature Probe in CH 1, proceed directly to Step 5. If it does not, continue with this step to set up your sensor manually.

- Press  to select CH 1.

- Select TEMPERATURE from the SELECT SENSOR menu.

- Select the correct Temperature Probe (in °C) from the TEMPERATURE menu.

5.	Set up the calculator and interface for data collection.

- Use  and  to select MODE and press .

- Select TIME GRAPH from the SELECT MODE menu.

- Select CHANGE TIME SETTINGS from the TIME GRAPH SETTINGS menu.

- Enter 5 as the time between samples in seconds.

- Enter 60 as the number of samples. Data collection will last 300 seconds (5 minutes).

- Select OK to return to the setup screen.

- Select OK again to return to the main screen.

6.	Measure out 3.0 g of each of the test substances. Use and label a new piece of weighing paper for each substance.

7.	Use a 10 mL graduated cylinder to measure out 10 mL of room-temperature water into a clean 50 mL beaker.

8.	Collect temperature data.

- Place the Temperature Probe into the 50 mL beaker containing the 10 mL of water.

- Gently move the probe and note the temperature displayed at the top of the main screen.

- When the temperature stops changing, select START to begin data collection.

- Monitor the temperature for 15 seconds to establish the initial temperature of the water.

- Carefully add the solid ammonium chloride, NH4Cl, to the water. Stir gently with the Temperature Probe.

- When the temperature stops changing, press  to end data collection.

9.	Record the minimum and maximum temperatures.

- When data collection stops, a graph of temperature vs. time will be displayed. Use  to examine data points along the curve. As you move the cursor right or left, the time (X) and temperature (Y) values of each data point are displayed below the graph.

- Record the minimum and maximum temperatures (to the nearest 0.1°C).

- Press  to return to the main screen.

10.	Repeat Steps 7–9 for each of the remaining substances. Clean the probe after each run and place it into a 250 mL beaker containing room-temperature water to bring the probe back to room temperature.

Part II  Finding the Best Cold-Pack Mixture

11.	Make and test a plan for making the coldest temperature using 3.0 g of one of the solid substances and the best amount of water. Turn in a report that includes your procedure and results.

DATA

| Substance | Maximum temperature (°C) | Minimum temperature (°C) | Temperature change (°C) |
| --- | --- | --- | --- |
| Ammonium chloride (NH4Cl) |  |  |  |
| Citric acid (H3C6H5O7) |  |  |  |
| Potassium chloride (KCl) |  |  |  |
| Sodium bicarbonate (NaHCO3) |  |  |  |
| Sodium carbonate (Na2CO3) |  |  |  |

PROCESSING THE DATA

1.	In the space provided in the data table above, subtract to calculate the temperature changes. If the temperature went down, mark the answer with a down arrow (). If the temperature went up, mark the answer with an up arrow ().

2.	Which substance caused the greatest temperature decrease?

3.	Which substance is the most unsuitable for a cold pack? Explain.

4.	How did your Part II results compare with those of other student groups?

5.	Which factors other than cooling ability might be considered when choosing a substance for use in a cold pack?

EXTENSIONS

1.	Research the types of injuries that are treated with the use of a cold pack.

2.	Make a cold pack using your suggested substance, water, and Ziplock or other bags.