Yeast Beasts in Action

Yeasts are microorganisms. They can break hydrogen peroxide down into water and oxygen gas. In this experiment you will investigate yeast activity in acidic, neutral, and basic mixtures. You will observe yeast activity by measuring pressure caused by the oxygen gas they produce.

OBJECTIVES

In this experiment, you will

- Use a Pressure Sensor to measure pressure.

- Measure and compare yeast activity in acidic, neutral, and basic mixtures.

- Make a conclusion about yeast activity.

MATERIALS

| LabPro or CBL 2 interface | yeast suspension |
| --- | --- |
| TI graphing calculator | 3 test tubes |
| DataMate program | test tube rack |
| Vernier Gas Pressure Sensor | dropper |
| 1-hole stopper with tapered valve | acidic mixture (diet soft drink) |
| 10 mL graduated cylinder | neutral mixture (skim milk) |
| 3% hydrogen peroxide | basic mixture (stomach antacid) |

Figure 1

PROCEDURE

1.	Obtain and wear goggles.

2.	Plug the Pressure Sensor into Channel 1 of the LabPro or CBL 2 interface. Use the link cable to connect the TI graphing calculator to the interface. Firmly press in the cable ends.

3.	Turn on the calculator and start the DATAMATE program. Press  to reset the program.

4.	Set up the calculator and interface for the Pressure Sensor.

- Select SETUP from the main screen.

- If the calculator displays a Pressure Sensor set to kPa in CH 1, proceed directly to Step 5. If it does not, continue with this step to set up your sensor manually.

- Press  to select CH 1.

- Select PRESSURE from the SELECT SENSOR menu.

- Select the correct Pressure Sensor from the PRESSURE menu.

- Select the calibration listing for units of (KPA) from the PRESSURE SENSOR menu.

5.	Set up the calculator and interface for data collection.

- Use  and  to select MODE and press .

- Select TIME GRAPH from the SELECT MODE menu.

- Select CHANGE TIME SETTINGS from the TIME GRAPH SETTINGS menu.

- Enter 6 as the time between samples in seconds.

- Enter 20 as the number of samples. Data collection will last 120 seconds (2 minutes).

- Select OK to return to the setup screen.

- Select OK again to return to the main screen.

6.	Place three clean test tubes in a test tube rack and label them A, N, and B (for acid, neutral, and base). Add 3 mL of hydrogen peroxide and 3 mL of each mixture to its test tube, as directed in the table below.

| Mixture | mL Hydrogen peroxide | mL Mixture |
| --- | --- | --- |
| A (acid) | 3 | 3 |
| N (neutral) | 3 | 3 |
| B (base) | 3 | 3 |

7.	Using a clean dropper, add 2 drops of yeast to Test Tube A. Note: Be sure not to let the yeast fall against the side of the test tube.

Figure 2

8.	Insert the 1-hole stopper assembly into the test tube. Note: Firmly twist the stopper for an airtight fit.

9.	Swirl the tube to thoroughly mix its contents. Stand the test tube in the rack.

10.	Select START to begin data collection.

11.	Connect the free end of the plastic tubing to the connector in the rubber stopper as shown in Figure 2.

12.	Data will be collected for 120 seconds. Do not shake the test tube during data collection.

13.	Record the highest pressure for this run.

- When data collection is complete after 120 seconds, a graph of pressure vs. time will be displayed. Use  to examine data points along the curve. As you move the cursor right or left, the time (X) and pressure (Y) values of each data point are displayed below the graph.

- Record the highest pressure reading.

- Press  to return to the main screen.

14.	Disconnect the tubing from the rubber stopper and remove the rubber stopper from the test tube.

15.	Repeat Steps 7–14 using Test Tube N.

16.	Repeat Steps 7–14 using Test Tube B.

DATA

| Mixture | Highest pressure (kPa) |
| --- | --- |
| Acidic (diet soft drink) |  |
| Neutral (skim milk) |  |
| Basic (stomach antacid) |  |

PROCESSING THE DATA

1.	In which mixture was the yeast activity greatest? Explain.

2.	In which mixture was the yeast activity least? Explain.

3.	What can you conclude from the results of your experiment?

EXTENSIONS

1.	Prepare a bar graph of the results of your experiment.

2.	Observe yeast growing under a microscope.

3.	Use a pH system to measure the pH of each mixture used in this experiment.

4.	Design an experiment to find the best pH for yeast. Perform the experiment you designed.