Heart Rate and Exercise

Heart rates increase during exercise. The heart rates of physically fit people increase less during exercise than those of less fit people. The heart rates of physically fit people recover to their resting rates more rapidly. In this experiment, you will use a Heart Rate Monitor to measure your heart rate as you stand quietly for 100 seconds, step onto and off a stool for 100 seconds, and then stand quietly for 200 seconds. You will then analyze a graph of the results to determine your standing heart rate, your maximum heart rate during the 100 seconds of exercise, and your recovery time.

Important: Do not attempt this experiment if physical exertion will aggravate a health problem. Inform your teacher of any possible health problems that might be affected if you participate in this experiment.

OBJECTIVES

In this experiment, you will

- Use a Heart Rate Monitor to measure your heart rate.

- Determine the effect of exercise on your heart rate.

- Determine your recovery time.

- Compare your results with those of other students.

MATERIALS

| LabPro or CBL 2 interface | saline solution in a dropper bottle |
| --- | --- |
| TI graphing calculator | stepping stool, 40 cm high |
| DataMate program | timer |
| Vernier Hand-Grip Heart Rate Monitor of |  |
| Vernier Exercise Heart Rate Monitor |  |

PROCEDURE

1.	Plug the Hand-Grip or Exercise Heart Rate Monitor into Channel 1 of the LabPro or CBL 2 interface. Use the link cable to connect the TI graphing calculator to the interface. Firmly press in the cable ends.

2.	Turn on the calculator and start the DATAMATE program. Press  to reset the program.

3.	Set up the calculator and interface for the Exercise Heart Rate Monitor. Note: Both the Hand-Grip and Exercise Heart Rate Monitors use the same receiver and calibration information. Therefore, set up the experiment for an Exercise Heart Rate Monitor no matter which type you are using.

- Select SETUP from the main screen.

- If the calculator displays EX HEART RT (BPM) in CH 1, proceed directly to Step 4. If it does not, continue with this step to set up your sensor manually.

- Press  to select CH 1.

- Select HEART RATE from the SELECT SENSOR menu.

- Select EX HEART RATE (BPM) from the HEART RATE menu.

4.	Set up the calculator and interface for data collection.

- Use  and  to select MODE and press .

- Select TIME GRAPH from the SELECT MODE menu.

- Select CHANGE TIME SETTINGS from the TIME GRAPH SETTINGS menu.

- Enter 5 as the time between samples in seconds.

- Enter 80 as the number of samples. Data collection will last 400 seconds.

- Select OK to return to the setup screen.

- Select OK again to return to the main screen.

Using a Hand-Grip Heart Rate Monitor

Use the following instructions if you are using a Hand-Grip Heart Rate Monitor.

Figure 1

5.	Stand facing your table or lab bench. Grasp the handles of the Hand-Grip Heart Rate Monitor your hands. The fingertips of each hand should be placed on the reference areas of the handles. The left hand grip and the receiver are both marked with an alignment arrow. When collecting data, be sure that the arrow labels on each of these devices are in alignment (see Figure 1). The reception range of the plug-in receiver is 80−100 cm, or about 3 feet.

6.	Monitor the live readouts on the calculator screen to determine that the sensor is functioning correctly. There will be a 10–15 second delay while data are collected before the first heart rate is reported. The readings should be consistent and within the normal range of the individual, usually between 55 and 90 beats per minute.

7	If the readings appear unreasonable, reposition your fingers and check the sensor and receiver alignment.

8.	Once it is apparent that the sensor is functioning correctly, continue with Step 13.

Using an Exercise Heart Rate Monitor

Use the following instructions if you are using an Exercise Heart Rate Monitor.

9.	Depending upon your size, select a small or large size elastic strap. Secure one of the plastic ends of the elastic strap to the transmitter belt. It is important that the strap provide a snug fit of the transmitter belt.

| 10. Wet each of the electrodes (the two grooved rectangular areas on the underside of the transmitter belt) with 3 drops of saline solution. 11. Secure the transmitter belt against the skin directly over the base of the rib cage. The POLAR logo on the front of the belt should be centered. Adjust the elastic strap to ensure a tight fit. | Figure 1 |
| --- | --- |

12.	Take the receiver module of the Heart Rate Monitor in your right hand and stand quietly. Remember that the receiver must be within 80 cm of the transmitter in the Heart Rate Monitor belt.

Part I  Standing Heart Rate

13.	Once the subject has been standing quietly for about a minute, a partner should select START to begin monitoring heart rate. Another partner should begin timing at the same time. The subject should stand quietly for 100 seconds.

Part II  Step Test

14.	When 100 seconds have gone by, immediately begin to exercise as follows:

- Place your right foot on the top step of the stool.

- Place your left foot completely on the top step of the stool next to your right foot.

- Place your right foot back on the floor.

- Place your left foot completely on the floor next to your right foot.

- Each stepping cycle should take 3 seconds to complete.

- Continue the step test for a total of 100 seconds.

Part III  Recovery Time

15.	After 100 seconds of stepping (a total of exactly 200 seconds of data collection), stop stepping. Stand quietly for 200 seconds. Data collection will end automatically.

Part IV  Recording Data

16.	Examine the graph that is displayed after data collection ends. You should be able to recognize three parts of the graph that match with the standing, exercising, and recovery parts of the experiment.

17.	Determine the subject’s standing heart rate.

- Press  to return to the main screen.

- Select ANALYZE from the main screen.

- Select STATISTICS from the ANALYZE OPTIONS menu.

- Note the flattest part of the first 1/4 of the graph (up to about 100 seconds) that matches with the standing part of the experiment.

- Use  to move the cursor to the beginning of this flattest part and press .

- Move the cursor to the end of the flat part and press .

- Record the MEAN (average) heart rate (to the nearest whole bpm). This is the subject’s standing heart rate.

18.	Determine the subject’s maximum heart rate.

- Press  to return to the ANALYZE OPTIONS menu.

- Select RETURN TO MAIN SCREEN from the ANALYZE OPTIONS menu.

- Select GRAPH from the main menu.

- Use  to move the cursor to the highest point on the graph.

- Record the time (in s) and the heart rate (to the nearest whole bpm) displayed for this point.

19.	Determine the time when the subject’s standing heart rate returned to within 3 bpm of the standing rate.

- Starting at the highest point on the graph, use  to move the cursor to the right.

- Stop where the displayed heart rate first reads within 3 bpm of the Step 13 value.

- Record the time for this point.

20.	Sketch or print the graph as directed by your teacher.

21.	Repeat Steps 5–20 with other members of the team serving as subjects.

DATA

| Subject’s name |  |  |  |  |
| --- | --- | --- | --- | --- |
| Standing heart rate (bpm) |  |  |  |  |
| Maximum heart rate (bpm) |  |  |  |  |
| Time at maximum heart rate (s) |  |  |  |  |
| Time at return to within 3 bpm of standing rate (s) |  |  |  |  |
| Recovery time in seconds (s) |  |  |  |  |

PROCESSING THE DATA

1.	In the space provided in the data table, calculate the recovery time for each subject. (Subtract the time at maximum heart rate from the time when the rate returned to within 3 bpm of the standing rate.)

2.	Compare the standing heart rates of the members of your group.

3.	How does your maximum heart rate compare to other students in your group? Is this what you expected? How do you account for this?

4.	How does your recovery time compare to other students in your group? Is this what you expected? How do you account for this?

5.	Why would athletes need to work longer and harder before their heart rates were at the maximum value? Explain.

EXTENSION

1.	Survey your classmates to determine how much they exercise each week. Determine if there is a relationship between amount of exercise and recovery time for your class.