A Hot Hand

In this class, you will often work in a team using a calculator to investigate scientific concepts. You will measure the temperature of the palm of your hand and the palm temperatures of your teammates in this experiment. In the process, you will learn how to use EasyData, a calculator application you will be using throughout the school year, and how to use Temperature Probes. You will also get to know your teammates better.

OBJECTIVES

In this experiment, you will

- Use a Temperature Probe to measure temperature.

- Calculate temperature averages.

- Compare results.

MATERIALS

| TI-83 Plus or TI-84 Plus graphing calculator | beaker |
| --- | --- |
| EasyData application | water |
| EasyTemp or | paper towel |
| Temperature Probe and data-collection interface |  |

Figure 1

PROCEDURE

1.	Turn on the calculator and make sure it is on the home screen. Connect the Temperature Probe to the calculator. (This may require the use of a data-collection interface.)

2.	Follow these steps to setup the EasyData application.

- If EasyData is not already running, press . Use  to move down the list to EasyData and press .

- You are now at the Main screen of the application. Select  from the Main screen, and then select New to reset the application.

3.	Set up EasyData for data collection.

- Select  from the Main screen, then select Time Graph…

- Select  on the Time Graph Settings screen.

- Enter 1 as the time between samples in seconds.

- Select .

- Enter 60 as the number of samples and select . Data collection will last 60 seconds.

- Select  to return to the Main screen.

4.	Measure the temperature of the palm of your hand.

- Select  to begin data collection.

- Pick up the Temperature Probe and hold its tip in the palm of your hand as shown in Figure 1. Data collection will end when 60 seconds have gone by.

5.	Record your highest temperature.

- When data collection is complete, a graph of temperature vs. time will be displayed. Use  and  to examine data points along the curve. As you move the cursor right or left, the time (X) and temperature (Y) values of each data point are displayed above the graph.

- In your data table, record your highest temperature (to the nearest 0.1°C).

- Select  to return to the Main screen.

6.	Prepare the Temperature Probe for the next run.

- Cool the Temperature Probe by placing it into a beaker of room-temperature water until its temperature reaches the temperature of the water. The temperature of the probe is displayed at the top of the Main screen.

- Use a paper towel to dry the probe. Be careful not to warm the probe as you dry it.

7.	Repeat Steps 4–6 for each person in your team.

DATA

| Student name | Highest temperature |
| --- | --- |
|  | °C |
|  | °C |
|  | °C |
|  | °C |
| Team average | °C |

PROCESSING THE DATA

1.	Calculate your team average for the highest temperatures. Record the result in the data table above.

- How did the highest temperatures of your teammates compare?

3.	Who had the “hottest hand”?

EXTENSION

1.	Determine the class average for highest temperature.