Solar Homes

Alternative energy sources are energy sources other than the nonrenewable fossil fuels—coal, petroleum, and natural gas. Solar energy, or energy from the sun, is one energy source alternative. A passive solar heating system uses no pumps, fans, or mechanical devices. Insulation and heat storage are important factors in such a system. A thermal mass is a material that absorbs and stores heat. Thermal masses can keep a home from heating or cooling too fast. In this experiment, you will examine the effectiveness of a thermal mass. You will then use what you learn to design and build a model solar home.

OBJECTIVES

In this experiment, you will

- Measure temperature.

- Find the relationship between thermal mass and the ability of a solar home to retain heat.

- Design, build, and test a model solar home.

MATERIALS

| TI-83 Plus or TI-84 Plus graphing calculator | empty bottle with screw-on cap |
| --- | --- |
| EasyData application | room-temperature water |
| data-collection interface | lamp with 100 watt bulb |
| 2 Temperature Probes | 2 pieces of cardboard to cover the |
| 2 model solar homes | model solar home windows |
| masking tape | clock |

Figure 1

PROCEDURE

Part I

1.	Obtain two model solar homes and position them 20 cm apart. The window sides should face each other as shown in Figure 1.

2.	Fill a bottle with room temperature water. This will be the thermal mass for Part I. Firmly tighten the bottle cap and lay the filled bottle inside one of the model solar homes. Leave the other model home empty. Tape both model homes shut.

3.	Position a lamp to shine down between the model solar homes as shown in Figure 1. The lamp bulb should be 10 cm above the tabletop. It should be the same distance from each of the model homes. Do not turn on the lamp until instructed to do so in Step 10.

4.	Position Temperature Probe 1 in the model solar home with no thermal mass and Temperature Probe 2 in the model solar home with the thermal mass. In both cases, pass half of the probe through the hole provided. Make sure the probe is not in direct light from the lamp.

5.	Turn on the calculator and connect it to the data-collection interface. Connect Temperature Probe 1 to Channel 1 of the interface and Temperature Probe 2 to Channel 2.

6.	Set up EasyData for data collection.

- Start the EasyData application, if it is not already running.

- Select  from the Main screen, and then select New to reset the application.

- Select  from the Main screen, then select Time Graph…

- Select  on the Time Graph Settings screen.

- Enter 240 as the time between samples in seconds.

- Select .

- Enter 20 as the number of samples and select . Data collection will last 4800 seconds (80 minutes).

- Select  to return to the Main screen.

7.	You will be collecting data for 80 continuous minutes. During the first 40 minutes, the light will be on. After 40 minutes have passed, the light will be turned off and the windows of the model solar homes will be covered. Data will be collected for 40 more minutes with the light off.

8.	Select  to begin data collection. Then, switch on the light bulb.

9.	After 40 minutes, turn the light off and cover the window of each model solar home with a piece of cardboard.

10.	Determine the maximum temperature reached in each of the model solar homes.

- When data collection is complete, a graph of temperature vs. time will be displayed and the cursor will be on the curve for Probe 1(no thermal mass). Initially the cursor will be on the Probe 1 plot (the line without markers). Use  and  to examine data points along the curve for Probe 1. As you move the cursor right or left, the time (X) and temperature (Y) values of each data point are displayed above the graph.

- Move the cursor to the highest point on the Probe 1 curve.

- Record the Probe 1 maximum temperature (to the nearest 0.1°C).

- Use  to move the cursor to the curve for Probe 2. Examine data points along the curve for Probe 2 (with thermal mass, plotted with square markers).

- Record the Probe 2 maximum temperature (to the nearest 0.1°C).

11.	Using the Step 10 procedure, determine and record the temperature at 80 minutes (4800 seconds) for each probe.

12.	Sketch or print the graph as directed by your teacher.

DATA

|  | Probe 1 No thermal mass | Probe 2 With thermal mass |
| --- | --- | --- |
| Maximum temperature | °C | °C |
| Temperature at 80 minutes | °C | °C |
| Temperature change | °C | °C |

PROCESSING THE DATA

1.	In the space provided in the data table, subtract to find the temperature changes. Which model solar home cooled more?

2.	How are the “no thermal mass” and the “with thermal mass” curves similar? How are they different?

3.	Which model solar home heated more slowly?

4.	Which model solar home cooled more slowly?

5.	What effect does the thermal mass have on the heating and cooling of a model solar home?

6.	What are some advantages of heating a home with solar energy?

7.	What is a disadvantage of heating a home with solar energy?

Part II  Design Problem

1.	Using the information you gained in this and other experiments, design and build a model solar home that cools more slowly than the one you tested in Part I.

- Begin with a model solar home like the one used in Part I.

- You may add no more than 3 cm to the thickness of the walls.

- You may use no more than 600 mL of thermal mass.

- Your home must have a window with an area of at least 150 cm2.

- When testing your model, first use a lamp to raise the inside temperature to equal the maximum temperature inside the “with thermal mass” model in Part I. Then turn off the lamp, cover the window, and monitor temperature for 40 minutes as in Part I.

- Compare the results with your Part I results and the Part II results of your classmates.

- Explain why you chose the materials you did.

EXTENSIONS

1.	Run the experiment for two or more consecutive “daily” cycles of four hours or longer.

2.	Design an experiment to test other types of thermal mass, such as stones or phase-change materials.

3.	Design an experiment to test other variables affecting a solar home, such as color, window material, window size, and insulation type.