How Low Can You Go?

In this experiment, you will first find the normal melting temperature of ice. Then by adding salt to melting ice, you will see the effect of salt on the melting temperature of ice. You will next make and test a plan for reaching the coldest temperature using water, ice, and salt. Finally, you will have a contest to see which group can make their mixture reach the lowest temperature.

OBJECTIVES

In this experiment, you will

- Measure temperature.

- Find the normal melting temperature of ice.

- See the effect of adding salt on ice’s melting temperature.

- Plan and test an experiment to find the salt and ice combination that will make the coldest temperature.

- Compete with other groups trying to reach the coldest temperature.

- Apply the results of the experiment.

MATERIALS

| TI-83 Plus or TI-84 Plus graphing calculator | 250 mL beaker |
| --- | --- |
| EasyData application | crushed ice |
| EasyTemp or | salt |
| Temperature Probe and data-collection interface | balance |

Figure 1

PROCEDURE

Part I  The Melting Temperature of Ice

1.	Turn on the calculator. Connect the Temperature Probe to the calculator. (This may require the use of a data-collection interface.)

2.	Set up EasyData for data collection.

- Start the EasyData application, if it is not already running.

- Select  from the Main screen, and then select New to reset the application.

- Select  from the Main screen, then select Time Graph…

- Select  on the Time Graph Settings screen.

- Enter 10 as the time between samples in seconds and select .

- Enter 60 as the number of samples and select .

- Select  to return to the Main screen.

3.	Put 100 mL of crushed ice into a 250 mL beaker.

4.	Place the Temperature Probe into the ice, select  to begin data collection.

5.	Hold the beaker in one hand and the Temperature Probe with your other hand. Stir the ice until a steady temperature is reached. Record this temperature. It is the normal melting temperature of ice.

Part II  The Effect of Salt on Ice’s Melting Temperature

6.	Add 5.0 grams of salt to the ice water and stir. Continue stirring until the temperature stops dropping.

7.	Record the lowest temperature reached.

- When the temperature stops dropping, select  to end data collection.

- A graph of temperature vs. time will be displayed. Use  and  to examine data points along the curve. As you move the cursor right or left, the time (X) and temperature (Y) values of each data point are displayed above the graph.

- Record the lowest temperature reached (to the nearest 0.1°C).

- Select  to return to the Main screen.

Part III  Finding the “Coldest” Mixture

8.	Make and test a plan for finding the coldest possible temperature using 5.0 grams of salt and the materials used in Parts I and II. Outline your plan in the Contest Plan section below.

Part IV  The Coldest-Temperature Contest

9.	Set up the equipment as in Part I.

10.	Put the amounts of water and ice you found to be best in Step 8 into the 250 mL beaker. Note: All student groups should do this part of the experiment at the same time.

11.	Get 5 grams of salt from your teacher. Add this salt to the water and ice. Place the Temperature Probe into the water, ice, and salt mixture and then select , then  to overwrite the previous data and begin data collection.

12.	Stir until your coldest temperature is reached. Then select  to end data collection.

DATA

| Normal melting temperature of ice | °C |
| --- | --- |
| Coldest salt and ice-water temperature (Part II) | °C |

CONTEST PLAN

PROCESSING THE DATA

1.	How does the melting temperature of water (ice) compare with its freezing temperature? Hint: See your results for Experiment 14, “Freezing Temperature of Water.”

2.	What is the effect of adding salt on ice’s melting temperature?

3.	If you had a chance to repeat the contest, what would you do differently?

4.	List some uses for the ideas studied in this experiment.