Friction

Friction is a force that resists motion. It involves objects in contact with each other, and it can be either useful or harmful. Friction helps when you want to slow or stop a bicycle, but it is harmful when it causes wear on the parts of a machine. In this activity, you will study the effects of surface smoothness and the nature of materials in contact on sliding friction. You will use a Force Sensor to measure frictional force, in Newtons (N), as you pull a block across different surfaces.

OBJECTIVES

In this experiment, you will

- Measure sliding friction.

- Measure friction between a wooden block and smooth-surface wood.

- Measure friction between a wooden block and rough-surface wood.

- Make predictions about other surfaces.

- Test your predictions.

MATERIALS

| TI-83 Plus or TI-84 Plus graphing calculator | paper clip |
| --- | --- |
| EasyData application | wood with smooth surface |
| data-collection interface | wood with rough surface |
| Vernier Force Sensor | sandpaper |
| wooden block (with a hook) |  |

Figure 1

PROCEDURE

Part I  Smooth and Rough Surfaces

1.	Set up the Force Sensor.

- Turn on the calculator.

- Set the Force Sensor range switch to 10 N.

- Connect the Force Sensor, data-collection interface, and calculator.

2.	Set up EasyData for data collection.

- Start the EasyData application, if it is not already running.

- Select  from the Main screen, and then select New to reset the application.

- Select  from the Main screen, then select Time Graph…

- Select  on the Time Graph Settings screen.

- Enter 0.05 as the time between samples in seconds.

- Select .

- Enter 60 as the number of samples and select . Data collection will last 3 seconds.

- Select  to return to the Main screen.

3.	Zero the Force Sensor.

- Lay the Force Sensor on the tabletop in the position shown in Figure 1.

- Select  from the Main screen, and then select Zero…

- Select  to zero the Force Sensor Sensor.

4.	Get a wooden block (with a hook on one end). Partly straighten a paper clip—leaving a hook at each end. Use the paper clip to attach the wooden block to the Force Sensor.

5.	Slowly pull the wooden block across a piece of wood with a smooth surface. Hold the Force Sensor by its handle and pull it to you, as demonstrated by your teacher. The Force Sensor should be held parallel to and about 1 cm above the surface. Once the wooden block is moving at a steady rate, select  to begin data collection.

6.	Determine and record the force used to pull the block.

- When data collection is complete, a graph of force vs. time will be displayed. Select , and then select Statistics…

- Before moving the cursor, select  to set the left bound at 0 s.

- The cursor will move to the right edge of the graph. Ensure it is on the last point and select  to set the right bound.

- Record the mean (average) force (in N).

- Select  to return to the graph.

- Select  to return to the Main screen.

7.	Repeat Steps 5–6 as you pull the block over a piece of wood with a rough surface. Note: After selecting , select  to start data collection.

Part II  Predicting Friction

8.	You will measure friction as the block is pulled across your desktop, the floor, and sandpaper. In the space provided in the data table below, predict the order of friction for these surfaces—from lowest to highest.

9.	Repeat Steps 5–6 for each of the surfaces.

DATA

| Part I Smooth and rough surfaces |  |  |
| --- | --- | --- |
| Surface | Smooth wood | Rough wood |
| Force (N) |  |  |

| Part II Predicting friction |  |  |
| --- | --- | --- |
| Predicted order of values for desktop, floor, and sandpaper |  |  |
| (Lowest) |  | (Highest) |

| Surface | Desktop | Floor | Sandpaper |
| --- | --- | --- | --- |
| Force (N) |  |  |  |

PROCESSING THE DATA

1.	What is the effect of surface roughness on friction?

2.	How did you decide the order of your predictions in Part II?

3.	How good were your predictions? Explain.

4.	Give two examples of situations where friction is helpful.

5.	Give two examples of situations where it is best to reduce friction.

6.	Summarize the results of this experiment.

EXTENSIONS

1.	Test the friction of other surfaces, such as glass, metals, rubber, and different fabrics.

2.	Investigate how frictional force varies with contact area and mass.

3.	Design an experiment to test methods of reducing friction.