Velocity

Speed and velocity are rates. They tell us how much distance is covered in a unit of time. Velocity can be expressed by the formula

v = d/t

where v = velocity or speed (in m/s), d = distance traveled (in meters), and t = time (in sec). In this experiment, you will study the velocity of a car after it is released from different points on a ramp. A Motion Detector will be used to measure velocity.

OBJECTIVES

In this experiment, you will

- Measure velocity using a Motion Detector.

- Calculate average velocities.

- Determine the relationship between velocity and release point.

MATERIALS

| TI-83 Plus or TI-84 Plus graphing calculator | several books |
| --- | --- |
| EasyData application | meter stick |
| CBR 2 or Go! Motion and direct calculator cable | masking tape |
| or Motion Detector and data-collection interface | car |
| 1.8 m board | small index card |

Figure 1

PROCEDURE

1.	Prepare the track.

- Set up a ramp on books as shown in Figure 1. The high end of the ramp should be 45 cm above the floor.

- Place a meter stick down the center of the ramp. The 0 cm mark on the meter stick should be at the very bottom of the ramp. Tape the meter stick to the ramp in two places. The meter stick will serve as a guide rail for your car.

- Fasten the Motion Detector at the top and center of the ramp as shown in Figure 1.

2.	Use tape to fasten a small index card to the back of the car you will be using. The card acts as a reflective surface for the Motion Detector.

3.	Connect the Motion Detector.

- Open the pivoting head of the Motion Detector.

- If the Motion Detector has a sensitivity switch, set it to Normal.

- Turn on the calculator and connect it to the Motion Detector. (This may require the use of a data-collection interface.)

4.	Set up EasyData for data collection.

- Start the EasyData application, if it is not already running.

- Select  from the Main screen, and then select New to reset the application.

- Select  from the Main screen, then select Time Graph…

- Select  on the Time Graph Settings screen.

- Enter 0.2 as the time between samples in seconds.

- Select .

- Enter 15 as the number of samples and select . Data collection will last 3 seconds.

- Select  to return to the Main screen.

5.	Collect data.

- Place your car on the ramp with its front wheels at the 40 cm line.

- Select . Release the car after you hear a sound coming from the Motion Detector.

6.	Determine the velocity of the car.

- After data collection stops, a graph of distance versus time will be displayed. To view the velocity versus time graph, select , and then select Vel(m/s) vs Time.

- Use  and  to examine data points along the graph. As you move the cursor right and left, the time (X) and velocity (Y) values of each data point are displayed above the graph. The highest point on the graph corresponds to the highest velocity of the car. Record this highest velocity in the data table. Round to the nearest 0.01 m/s. (In the example to the right, the highest velocity is 1.52 m/s.)

- Select  to return to the Main screen.

7.	Repeat Steps 5 and 6 two more times. Note: After selecting , select  to overwrite the previous data and begin data collection.

8.	Repeat Steps 5–7 at distances of 70 cm and 100 cm.

DATA

|  | Velocity (m/s) |  |  |  |
| --- | --- | --- | --- | --- |
| Release point | Trial 1 | Trial 2 | Trial 3 | Average |
| 40 cm |  |  |  |  |
| 70 cm |  |  |  |  |
| 100 cm |  |  |  |  |

PROCESSING THE DATA

1.	Calculate the average velocity for each release point. Show your work here. Record the results in the data table.

2.	Make a graph of your data. Plot Release Point (in cm) on the horizontal or x-axis and Average Velocity (in m/s) on the vertical or y-axis.

3.	What happens to the velocity of the car as you release it from higher points?

4.	How does friction affect the velocity of your car?

5.	How could you improve your car to make it roll faster?

6.	Describe one way you could change the ramp to make the car go faster, without changing the release points.

EXTENSIONS

1.	Repeat the experiment with ramps of different heights.

2.	Design, build, and test a different car.