Crash Dummies

What happens to people in automobile collisions is important information for car manufacturers. Crash dummies are often used in collision studies. Velocity is one of the most important factors in such collision studies. In this experiment, you will study the relationship between car velocity and the distance a “crash dummy” is thrown during a collision. A Motion Detector will be used to measure car velocity. You will use a ruler to measure the distance a crash dummy is thrown.

OBJECTIVES

In this experiment, you will

- Measure velocity.

- Measure distances an action figure is thrown.

- Calculate averages.

- Graph the results.

- Make conclusions from your results.

MATERIALS

| computer | masking tape |
| --- | --- |
| Vernier computer interface | car |
| Motion Detector | 2 small index cards |
| 1.8 m board | small action figure (10-12 cm tall) |
| several books | ruler |
| meter stick |  |

Figure 1

Figure 2

PROCEDURE

1.	Prepare the ramp.

- Set up a ramp on books as shown in Figure 1. The high end of the ramp should be 45 cm above the floor.

- Place a meter stick down the center of the ramp. The 0 cm mark on the meter stick should be at the very bottom of the ramp. Tape the meter stick to the ramp in two places. The meter stick will serve as a guide rail for your car.

- Use tape to mark a distance 30 cm from the bottom end of the ramp. Place the bound edge of a heavy book on the 30 cm line where it will be struck head on by a car rolling off the ramp.

- Fasten the Vernier Motion Detector at the top and center of the ramp as shown in Figure 1. Connect the Motion Detector. If your Motion Detector has a sensitivity switch, set it to Track.

2.	Prepare a car for data collection.

- Tape a card to the back of a car. This card serves as a reflector for the Motion Detector.

- If your car does not have a seat for a crash dummy, fold another card in half. Fold each side up as shown in Figure 2. Tape this folded card to the car to serve as a seat for the crash dummy.

- Seat the action figure (crash dummy) in the car.

3.	Start the Vernier data-collection program and open the file “36 Crash Dummies” from the Middle School Science with Vernier folder.

4.	Collect data.

- Place something heavy on the book to keep it from moving.

- Place your car on the ramp with its front wheels at the 40 cm line.

- Click . Release the car after you hear a sound coming from the Motion Detector. Note: The car should crash into the book.

- When data collection has ended, click the Statistics button, . Record the maximum velocity in Table 1.

- Measure the distance the crash dummy was thrown (to the nearest cm). Record this value in Table 2. Measure from the seat of the car to the seat of the action figure.

5.	Repeat Step 4 two more times.

6.	Repeat Steps 4 and 5 at distances of 60 cm and 80 cm.

DATA

| Table 1: Velocity (m/s) |  |  |  |
| --- | --- | --- | --- |
| Release point | 40 cm | 60 cm | 80 cm |
| Trial 1 |  |  |  |
| Trial 2 |  |  |  |
| Trial 3 |  |  |  |
| Average |  |  |  |

| Table 2: Distance Thrown (cm) |  |  |  |
| --- | --- | --- | --- |
| Release point | 40 cm | 60 cm | 80 cm |
| Trial 1 |  |  |  |
| Trial 2 |  |  |  |
| Trial 3 |  |  |  |
| Average |  |  |  |

#### PROCESSING THE DATA

1.	Calculate the average velocity for each release point. Show your work here. Record the results in the Table 1.

2.	Calculate the average distance that the crash dummy was thrown for each release point. Show your work here. Record the results in the Table 2.

3.	Graph your results using the average values. Plot Velocity (in m/s) on the horizontal or x-axis and Distance Thrown (in cm) on the vertical or y-axis.

4.	What happened to the velocity of your car as it was released from higher release points?

5.	How did increased velocity affect the distance that your crash dummy was thrown?

6.	Do you think there would be more serious injuries in a high-speed or a low-speed collision? Explain.

7.	What is the purpose of seat belts?

EXTENSIONS

1.	Repeat the experiment with your car crashing into a moveable car instead of a book. Explain any differences in your results.

2.	Fasten the crash dummy into your car using a seat belt. Collect data while releasing the car from the 8 cm release point. Explain your results.