Diffusion: How Fast?

Diffusion is a process that allows particles to move from where they are more concentrated to where they are less concentrated. This process allows particles to move through a cell membrane. Diffusion enables cells to get food and get rid of waste.

A Conductivity Probe can measure the concentration of ions in a solution. A salt solution contains ions. In this experiment, you will use a Conductivity Probe to measure the diffusion of salt through a membrane. You will also study the effect of salt concentration on diffusion rate.

OBJECTIVES

In this experiment, you will

- Measure conductivity.

- Measure the diffusion of salt through a membrane.

- See the effect of salt concentration on diffusion rate.

MATERIALS

| LabQuest | 400 mL beaker |
| --- | --- |
| LabQuest App | dialysis tubing |
| Vernier Conductivity Probe | dropper |
| 5% and 10% saltwater | scissors |
| ring stand | stirring rod |
| utility clamp | 2 dialysis tube clamps or dental floss |
| 50 mL beaker | distilled water |

Figure 1

PROCEDURE

Part I  Measuring Diffusion

1.	Set the Conductivity Probe on the 0–2000 µS/cm position. Connect the Conductivity Probe to LabQuest and choose New from the File menu. If you have an older sensor that does not auto-ID, manually set up the sensor.

2.	On the Sensor screen, tap Rate. Change the data-collection rate to 0.5 samples/second and the data-collection length to 120 seconds.

3.	Get a wet dialysis tube and a dialysis tube clamp or a short length of dental floss. Using the clamp or floss, tie one end of the tube closed about 1 cm from the end, as in Figure 2.

Figure 2

4.	Put 5% saltwater into the dialysis tubing.

- Get about 15 mL of 5% saltwater in the 50 mL beaker.

- Use a dropper to transfer about 10 mL of the 5% saltwater into the tube. Note: To open the tube, you may have to rub it between your fingers.

- Tie off the top of the tube with a clamp or a new length of dental floss. Try not to allow any air to stay in the tube.

- Wash the outside of the tubing with distilled water to wash away any saltwater.

5.	Put 250 mL of distilled water into a 400 mL beaker. Position the Conductivity Probe in the distilled water as shown in Figure 1.

6.	Put the dialysis tube into the water. Make sure it is completely covered with water. Important: The Conductivity Probe and the tube must be the same distance apart in both parts of the experiment.

7.	After stirring the water for 15 seconds, start data collection. Stir the water slowly and continuously for the next 2 minutes. Data collection will end automatically after 2 minutes.

8.	Record your beginning and final conductivity readings

- When data collection ends, a graph of conductivity vs. time will be displayed. To examine the data pairs on the displayed graph, tap any data point. As you tap each data point, the conductivity values of each data point are displayed to the right of the graph.

- Record your beginning and final conductivity readings in your data table.

Part II  The Effect of Salt Concentration on Diffusion

9.	Use a tissue to carefully blot the Conductivity Probe dry.

10.	Prepare the dialysis tubing for reuse.

- Remove one of the clamps. If the tubing is tied off with floss, use scissors to carefully cut one of the dental floss strings. Discard the floss.

- If you cut the tubing, replace it.

- Empty all of the liquid out of the dialysis tube. With your fingers, squeeze out any remaining liquid.

- Wash the outside of the tubing with tap water to wash away any saltwater.

11.	Repeat Steps 4–8 using 10% saltwater.

DATA

| Salt concentration | 5% | 10% |
| --- | --- | --- |
| Final conductivity (µS/cm) |  |  |
| Beginning conductivity (µS/cm) |  |  |
| Conductivity increase (µS/cm) |  |  |

PROCESSING THE DATA

1.	Describe what happened in this experiment.

2.	What evidence is there that salt diffused through the membrane?

3.	From which saltwater solution did salt diffuse faster? How could you tell?

EXTENSION

1.	Design an experiment to find the effect of temperature on the diffusion of salt. Perform the experiment you designed.