Reflectivity of Light

Light is reflected differently from various surfaces and colors. In this experiment, you will be measuring the percent reflectivity of various colors. You will measure reflection values from paper of the various colors using a Light Sensor and then compare these values to the reflection value of aluminum foil. You will then calculate percent reflectivity using the relationship:

% Reflectivity  =  X 100

OBJECTIVES

In this experiment, you will

- Use a Light Sensor to measure reflected light.

- Calculate percent reflectivity of various colors.

- Make conclusions using the results of the experiment.

MATERIALS

| LabPro interface | white paper |
| --- | --- |
| Palm handheld | black paper |
| Data Pro program | 2 other pieces of colored paper |
| Light Sensor | ring stand |
| aluminum foil | utility clamp |

Figure 1

PROCEDURE

1.	Use a utility clamp and ring stand to fasten a Light Sensor 5 cm above a piece of colored paper as shown in Figure 1. The classroom lights should be on. Note: If you are using a Vernier Light Sensor, set it to the 0-6000 lux position.

2.	Plug the Light Sensor into Channel 1 of the LabPro interface. Connect the handheld to the LabPro using the interface cable. Firmly press in the cable ends.

3.	Press the power button on the handheld to turn it on. To start Data Pro, tap the Data Pro icon on the Applications screen. Choose New from the Data Pro menu or tap  to reset the program.

4.	Set up the handheld and interface for the Light Sensor.

- If the handheld displays LIGHT or LIGHT(lux) in CH 1, proceed directly to Step 5. If it does not, continue with this step to set up your sensor manually.

- On the Main screen, tap .

- Tap  to select Channel 1.

- Press the Scroll buttons on the handheld to scroll through the list of sensors.

- Select the correct Light Sensor (LIGHT-TI CBL if you are using a TI Light Sensor or LIGHT 6000(lux) if you are using a Vernier Light Sensor) from the list of sensors.

- Tap  to return to the Main screen.

5.	The light readings are displayed on the main screen. When the reading stabilizes, record the color and the reflected light value in your data table.

6.	Make and record readings for aluminum, black, white, and two other colors.

DATA

| Color | Aluminum | Black | White | __________ | __________ |
| --- | --- | --- | --- | --- | --- |
| Reflection value |  |  |  |  |  |
| Percent reflectivity | 100% | % | % | % | % |

PROCESSING THE DATA

1.	Calculate the percent reflectivity of each color using the formula given in the introduction. Show your work in the data table above.

2.	Which color, other than aluminum, has the highest reflectivity?

3.	Which color has the lowest reflectivity?

4.	What surfaces might give a planet a high reflectivity? Explain.

5.	Does the planet Earth have high reflectivity? Why or why not?

EXTENSION

1.	Design an experiment to test the reflectivity of sand, soil, water, and other materials. Perform the experiment you designed.