Solar Homes

Alternative energy sources are energy sources other than the nonrenewable fossil fuels—coal, petroleum, and natural gas. Solar energy, or energy from the sun, is one energy source alternative. A passive solar heating system uses no pumps, fans, or mechanical devices. Insulation and heat storage are important factors in such a system. A thermal mass is a material that absorbs and stores heat. Thermal masses can keep a home from heating or cooling too fast. In this experiment, you will examine the effectiveness of a thermal mass. You will then use what you learn to design and build a model solar home.

OBJECTIVES

In this experiment, you will

- Measure temperature.

- Find the relationship between thermal mass and the ability of a solar home to retain heat.

- Design, build, and test a model solar home.

MATERIALS

| LabPro interface | empty bottle with screw-on cap |
| --- | --- |
| Palm handheld | room-temperature water |
| Data Pro program | lamp with 100 watt bulb |
| 2 Temperature Probes | 2 pieces of cardboard to cover the model |
| 2 model solar homes | solar home windows |
| masking tape | clock |

Figure 1

PROCEDURE

Part I

1.	Obtain two model solar homes and position them 20 cm apart. The window sides should face each other as shown in Figure 1.

2.	Fill a bottle with room temperature water. This will be the thermal mass for Part I. Firmly tighten the bottle cap and lay the filled bottle inside one of the model solar homes. Leave the other model home empty. Tape both model homes shut.

3.	Position a lamp to shine down between the model solar homes as shown in Figure 1. The lamp bulb should be 10 cm above the tabletop. It should be the same distance from each of the model homes. Do not turn on the lamp until instructed to do so in Step 10.

4.	Position Temperature Probe 1 in the model solar home with no thermal mass and Temperature Probe 2 in the model solar home with the thermal mass. In both cases, pass half of the probe through the hole provided. Make sure the probe is not in direct light from the lamp.

5.	Plug Temperature Probe 1 into Channel 1 of the LabPro interface. Plug Temperature Probe 2 into Channel 2. Connect the handheld to the LabPro using the interface cable. Firmly press in the cable ends.

6.	Press the power button on the handheld to turn it on. To start Data Pro, tap the Data Pro icon on the Applications screen. Choose New from the Data Pro menu or tap  to reset the program.

7.	Set up the handheld and interface for two Temperature Probes.

- On the Main screen, tap .

- If the handheld displays TEMP(C) in CH 1 and CH 2, proceed directly to Step 8. If it does not, continue with this step to set up your sensors manually.

- Tap  to select Channel 1.

- Press the Scroll buttons on the handheld to scroll through the list of sensors.

- Select the correct Temperature Probe (in °C) from the list of sensors.

- Tap  to select Channel 2.

- Press the Scroll buttons on the handheld to scroll through the list of sensors.

- Select the correct Temperature Probe (in °C) from the list of sensors.

8.	Set up the handheld and interface for data collection.

- While still on the Setup screen, tap .

- Enter “240” as the time between samples in seconds, using the onscreen keyboard (tap “123”) or using the Graffiti writing area.

- Enter “20” as the number of samples. (Data will be collected for 80 minutes.)

- Tap  twice to return to the Main screen.

9.	You will be collecting data for 80 continuous minutes. During the first 40 minutes, the light will be on. After 40 minutes have passed, the light will be turned off and the windows of the model solar homes will be covered. Data will be collected for 40 more minutes with the light off.

10.	Tap  to begin data collection. After you hear a beep, switch on the light bulb.

11.	After 40 minutes, turn the light off and cover the window of each model solar home with a piece of cardboard. Data collection will end when 4800 seconds (80 minutes) have passed.

12.	Determine the maximum temperature reached in each of the model solar homes.

- After data collection is complete, a graph of temperature vs. time will be displayed. On the displayed graph of temperature vs. time, each point for Probe 1 is plotted with an open square, and each point for Probe 2 with a filled square.

- To examine the data pairs on the displayed graph, tap  or any data point. As you move the examine line, the temperature values of both Probe 1 (top value) and Probe 2 (lower value) are displayed to the right of the graph.

- Move the examine line to identify the maximum temperatures reached during data collection for both Probe 1 and Probe 2. Record these values to the nearest 0.1°C.

13.	Using the Step 12 procedure, determine and record the temperature at 80 minutes (4800 seconds) for each probe.

14.	Sketch or print the graph as directed by your teacher.

DATA

|  | Probe 1 No thermal mass | Probe 2 With thermal mass |
| --- | --- | --- |
| Maximum temperature | °C | °C |
| Temperature at 80 minutes | °C | °C |
| Temperature change | °C | °C |

PROCESSING THE DATA

1.	In the space provided in the data table, subtract to find the temperature changes. Which model solar home cooled more?

2.	How are the “no thermal mass” and the “with thermal mass” curves similar? How are they different?

3.	Which model solar home heated more slowly?

4.	Which model solar home cooled more slowly?

5.	What effect does the thermal mass have on the heating and cooling of a model solar home?

6.	What are some advantages of heating a home with solar energy?

7.	What is a disadvantage of heating a home with solar energy?

Part II  Design Problem

1.	Using the information you gained in this and other experiments, design and build a model solar home that cools more slowly than the one you tested in Part I.

- Begin with a model solar home like the one used in Part I.

- You may add no more than 3 cm to the thickness of the walls.

- You may use no more than 600 mL of thermal mass.

- Your home must have a window with an area of at least 150 cm2.

- When testing your model, first use a lamp to raise the inside temperature to equal the maximum temperature inside the “with thermal mass” model in Part I. Then turn off the lamp, cover the window, and monitor temperature for 40 minutes as in Part I.

- Compare the results with your Part I results and the Part II results of your classmates.

- Explain why you chose the materials you did.

EXTENSIONS

1.	Run the experiment for two or more consecutive “daily” cycles of four hours or longer.

2.	Design an experiment to test other types of thermal mass, such as stones or phase-change materials.

3.	Design an experiment to test other variables affecting a solar home, such as color, window material, window size, and insulation type.