Ocean Floor Mapping

Oceanographers, marine geologists, and archeologists use sound to investigate objects below the surfaces of bodies of water. A signal is sent out and bounces back from a submerged surface. Scientists use the speed of sound in water and the time it takes for the signal to bounce back to calculate the depth of the object. Sonar is the name given to this system. The Vernier Motion Detector works in a similar manner. In this activity, you will use a Motion Detector to map objects on a simulated ocean floor.

OBJECTIVES

In this experiment, you will

- Use a Motion Detector to measure distances.

- Map simulated ocean floors.

- Analyze graphs to find the heights of objects on a simulated ocean floor.

MATERIALS

| LabPro interface | 1 m board |
| --- | --- |
| Palm handheld | masking tape |
| Data Pro program | 2 or more boxes |
| Vernier Motion Detector |  |

Figure 1

PROCEDURE

Part I  Ocean Floor 1

1.	Prepare the Motion Detector for data collection.

- Get the board that will act as the support for your Motion Detector.

- Tape or clamp the Motion Detector to one end of the board. Make sure that the round screen of the Motion Detector is not covered and is pointing downward.

- Place the board with the Motion Detector flat on your table as shown in Figure 1.

2.	Prepare the ocean floor for data collection.

- Place the box on the floor underneath the Motion Detector. Note: The Motion Detector must be at least 40 cm from the top of the box.

- Line up the Motion Detector so that when it is moved along the table edge it will pass over the box.

3.	Plug the Motion Detector into the DIG/SONIC 1 port of the LabPro interface. Use the link cable to connect the handheld to the interface. Firmly press in the cable ends.

4.	Press the power button on the handheld to turn it on. To start Data Pro, tap the Data Pro icon on the Applications screen. Choose New from the Data Pro menu or tap  to reset the program.

5.	Set up the handheld and interface for data collection.

- On the Main screen, tap .

- On the Setup screen, tap .

- Enter “0.25” as the time between samples in seconds, using the onscreen keyboard (tap “123”) or using the Graffiti writing area.

- Enter “60” as the number of samples. (Data collection will last 15 seconds.)

- Tap  twice to return to the Main screen.

6.	Collect distance data.

- Move the board to position the Motion Detector to the left of the box.

- When everything is ready, tap  to begin data collection.

- After you hear a beep, slowly slide the board across the tabletop so that the Motion Detector passes over and past the box.

7.	Determine and record the distance to the floor.

- After data collection stops, tap .

- Tap the Selection button, .

- Identify a flat portion of the graph that represents the floor. Tap the data point at the beginning of the flat portion. An arrow (>) is displayed on this line.

- Tap the data point at the end of the flat portion that represents the floor. An arrow (<) is displayed on this line.

- Tap .

- Record the Mean (average) distance (DIST) to the floor in meters in your data table.

- Tap  to return to the Analyze screen.

8.	Determine and record the distance to the box.

- Tap  to clear the previously selected region.

- Tap  again, to select a new region.

- Identify the flat portion of the graph that represents the box. Tap the data point at the beginning of the flat portion. An arrow (>) is displayed on this line.

- Tap the data point at the end of the flat portion that represents the box. An arrow (<) is displayed on this line.

- Tap .

- Record the Mean (average) distance (DIST) to the box in meters.

- Tap  to return to the Analyze screen.

9.	Sketch and label your graph.

Part II  Ocean Floor 2

10.	Prepare Ocean Floor 2.

- Set up two boxes in the shape of steps. The tallest box must be at least 40 cm from the Motion Detector.

- Repeat Steps 6-9. Be sure to record all three distances.

Part III  Hidden Ocean Floor

11.	Your teacher will have a hidden ocean floor for you to measure. Repeat Steps 6-9 for the concealed object or objects.

DATA

|  | Distance to floor (m) | Distance to box (m) | Box height (m) |
| --- | --- | --- | --- |
| Ocean floor 1 single box |  |  |  |
| Ocean floor 2 box 1 |  |  |  |
| Ocean floor 2 box 2 |  |  |  |
| Hidden ocean floor box 1 |  |  |  |
| Hidden ocean floor box 2 |  |  |  |
| Hidden ocean floor box 3 |  |  |  |

Graph Sketches

### Ocean Floor 1	Ocean Floor 2	Hidden Ocean Floor

PROCESSING THE DATA

1.	In the space provided in the data table above, find the height of each box. Do this by subtracting the distance to the box from the distance to the floor.

2.	Which was your best result? Why do you think it was better than your other results?

3.	How did the shape of your graph compare to the actual object(s) in each case? Explain.

4.	What factors might affect the accuracy of real ocean-floor mapping?

EXTENSIONS

1.	Try other hidden ocean-floor arrangements.

2.	Research the sonar process and compare it to what you did in this activity.