Boiling Temperature of Water

The physical properties of a pure substance can be used to identify the substance and distinguish it from other pure substances. Boiling temperature is one such physical property. This is the temperature at which a substance changes rapidly from its liquid state into a gas. Rapid formation of bubbles is evidence that the liquid is at its boiling temperature. In this experiment, you will study the boiling of water.

OBJECTIVES

In this experiment, you will

- Observe the boiling of water.

- Measure temperature.

- Analyze data.

- Use your data and graph to make conclusions about boiling.

- Determine the boiling temperature of water.

- Apply the concepts studied in a new situation.

MATERIALS

| LabPro interface | ring stand |
| --- | --- |
| Palm handheld | utility clamp |
| Data Pro program | hot plate |
| Temperature Probe | water |
| 250 mL beaker |  |

Figure 1

PROCEDURE

1.	Obtain and wear goggles. CAUTION: Handle hot water and hot equipment with care throughout the experiment.

2.	Plug the Temperature Probe into Channel 1 of the LabPro interface. Connect the handheld to the LabPro using the interface cable. Firmly press in the cable ends.

3.	Press the power button on the handheld to turn it on. To start Data Pro, tap the Data Pro icon on the Applications screen. Choose New from the Data Pro menu or tap  to reset the program.

4.	Set up the handheld and interface for the correct Temperature Probe.

- On the Main screen, tap .

- If the handheld displays TEMP(C) in CH 1, proceed directly to Step 5. If it does not, continue with this step to set up your sensor manually.

- Tap  to select Channel 1.

- Press the Scroll buttons on the handheld to scroll through the list of sensors.

- Select the correct Temperature Probe (in °C) from the list of sensors.

5.	Setup the handheld and interface for data collection.

- While still on the Setup screen, tap .

- Enter “60” as the time between samples in seconds, using the onscreen keyboard (tap “123”) or using the Graffiti writing area.

- Enter “20” as the number of samples. (Data will be collected for 20 minutes.)

- Tap  twice to return to the Main screen.

6.	Prepare the water sample.

- Arrange a hot plate next to the base of a ring stand as shown in Figure 1.

- Fill a 250 mL beaker 2/3 full with hot tap water.

- Place the 250 mL beaker on the hot plate. Turn the hot plate to the temperature setting suggested by your teacher.

- Use a utility clamp to suspend a Temperature Probe on the ring stand as shown in Figure 1. The tip of the probe should be 1-2 cm above the bottom of the beaker. CAUTION: Do not burn yourself or melt a probe wire with the hot plate!

7.	Tap  to begin data collection.

8.	Record your observations as the water is heated to its boiling temperature and boils.

9.	When the water has boiled with noticeable bubbling for six minutes, tap  to end data collection. Turn off the hot plate and remove the Temperature Probe from the boiling water. Allow the beaker, water, and hot plate to cool before handling them.

10.	Determine and record the boiling temperature of water.

- After data collection stops, tap .

- Tap the Selection button, .

- Identify the flat portion of the graph that represents boiling. Tap the data point at the beginning of the flat portion. An arrow (>) is displayed on this line.

- Tap on the data point at the end of the flat portion of the graph that represents boiling. An arrow (>) is displayed on this line.

- Tap .

- Record the Mean (average) temperature (to the nearest 0.1°C) in your data table. This is your value for the boiling temperature of water.

- Tap  to return to the graph.

11.	Sketch or print the graph as directed by your teacher. Label the boiling temperature on your graph.

OBSERVATIONS

DATA

Water’s boiling temperature	_____°C

PROCESSING THE DATA

1.	Describe your temperature vs. time graph.

2.	What happened to the temperature of the water as it was heated prior to boiling?

3.	What happened to the temperature of the water as it boiled?

| 4. The boiling temperature of isopropyl alcohol is 82°C. In the space to the right, sketch and label a graph for the boiling of isopropyl alcohol. Use a starting temperature of 20°C. Identify the boiling temperature on the graph. |  |
| --- | --- |

EXTENSION

1.	Determine the boiling temperatures of other liquids.