Get a Grip!

In this experiment, you will measure your gripping power. You will see if your gripping power changes as you grip an object over a period of time. You will also compare your gripping power with your classmates.

OBJECTIVES

In this experiment, you will

- Use a Gas Pressure Sensor to measure your gripping power.

- See which of your hands has the greater gripping power.

- Learn what happens to your gripping power as time goes by.

- Compare your gripping power with your classmates.

MATERIALS

| LabPro interface | plastic bottle |
| --- | --- |
| Palm handheld | 1-hole stopper with a tapered valve |
| Data Pro program | heavy-wall plastic tubing |
| Vernier Gas Pressure Sensor |  |

Figure 1

PROCEDURE

1.	Connect the plastic bottle to the Gas Pressure Sensor as shown in Figure 1.

2.	Plug the Gas Pressure Sensor into Channel 1 of the LabPro interface. Connect the handheld to the LabPro using the interface cable. Firmly press in the cable ends.

3.	Press the power button on the handheld to turn it on. To start Data Pro, tap the Data Pro icon on the Applications screen. Choose New from the Data Pro menu or tap  to reset the program.

4.	Setup the handheld and interface for data collection.

- From the Main screen, tap , and then tap .

- Enter “1” as the time between samples in seconds, using the onscreen keyboard (tap “123”) or using the Graffiti writing area.

- Enter “60” as the number of samples. (The length of the data collection will be 1 minute.)

- Tap  twice to return to the Main screen.

5.	Grip the bottle as hard as you can with one hand, then tap  to begin data collection. Keep gripping as hard as you can. Do not lean your hand or arm on anything.

6.	Determine and record your 0-60 s grip average.

- After data collection stops, tap .

- Tap .

- Record the Mean (average) grip (in kPa) for the 60 s period in your data table.

- Tap  to return to the Analyze screen.

7.	Determine and record your 0-10 s grip average.

- Tap the Selection button, .

- Tap on the vertical axis of the graph (0 s) to define the left boundary of the selection. An arrow (>) is displayed on this line.

- Tap on the data point at 10 s to define the right boundary. An arrow (<) is displayed on this line.

- Tap .

- Record the Mean (average) grip (in kPa) for the 0-10 s period in your data table.

- Tap  to return to the Analyze screen.

- Tap , to turn off Selection mode.

8.	Determine and record your 50-60 s grip average.

- Tap , ,or any data point to position the examine line on the data point at 50 s.

- Tap , to turn on Selection mode.

- Tap on the examine line at 50 s to define the left boundary. An arrow (>) is displayed on this line.

- Tap on the data point at 60 s to define the right boundary. An arrow (<) is displayed on this line.

- Tap .

- Record the Mean (average) grip (in kPa) for the 50-60 s period in your data table.

9.	Collect data for your other hand.

- Tap  to return to the Analyze screen.

- Tap , to turn off Selection mode.

- Repeat Steps 5-8 using your other hand.

DATA

| Your Results |  |  |
| --- | --- | --- |
|  | Left hand | Right hand |
| 0-60 s Grip average (kPa) |  |  |
| 0-10 s Grip average (kPa) |  |  |
| 50-60 s Grip average (kPa) |  |  |
| Difference between 0-10 s avg. and 50-60 s avg. |  |  |

| Group and Class Results |  |
| --- | --- |
| Name | Strong hand average for 0-60 s (kPa) |
| Group average |  |
| Class average |  |

PROCESSING THE DATA

1.	In the space provided in the data table, subtract to find the difference between your 0-10 s average and your 50-60 s average for each hand.

2.	Record the 0-60 s results for the other students in your group. Calculate and record your group average. Calculate and record the class average for 0-60 s.

3.	Which of your hands is stronger? Explain your decision.

4.	Did your gripping power increase or decrease during the 60 s period? Why did it change?

5.	How does your grip compare with the class average?

6.	What did you learn about your strength in this experiment? Were you surprised?

EXTENSION

1.	See if you can increase your gripping power with practice.