A Water Field Study

When finding water quality, there are many measurements that can be made. In this experiment, you will measure water temperature, conductivity, and pH. Water temperatures in streams can range from 0C in the winter to above 30C in the summer. Cooler water in a stream is generally considered healthier than warmer water. Problems generally occur when changes in water temperature are noted along one stream on the same day. Some sample data are listed in Table 1.

| Table 1: Water Temperatures of Selected Rivers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Site | Season | Temperature (°C) | Season | Temperature (°C) |
| Hudson River, Poughkeepsie, NY | Winter | 5 | Summer | 25 |
| Mississippi River, Memphis, TN | Winter | 7 | Summer | 29 |
| Rio Grande, El Paso, TX | Winter | 16 | Summer | 21 |
| Missouri River, Garrison Dam, ND | Winter | 3 | Summer | 14 |
| Willamette River, Portland, OR | Winter | 9 | Summer | 22 |

Conductivity values in lakes and streams are typically found to be in the range of 100 to 500 µS/cm. In areas of especially hard water or high salinity, conductivity values may be as high as 1000 µS/cm. Drinking water usually has conductivity in the 50 to 1000 µS/cm range. Some sample data are listed in Table 2.

| Table 2: Conductivity in Selected Rivers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Site | Season | Conductivity (µS/cm) | Season | Conductivity (µS/cm) |
| Hudson River, Poughkeepsie, NY | Spring | 180 | Fall | 238 |
| Mississippi River, Memphis, TN | Spring | 266 | Fall | 440 |
| Rio Grande, El Paso, TX | Spring | 1020 | Fall | 1220 |
| Sacramento River, Keswick, CA | Spring | 142 | Fall | 120 |
| Ohio River, Benwood, WV | Spring | 600 | Fall | 286 |

The best pH range for most aquatic organisms is pH 6.5 to 8.2. The pH values of streams and lakes are usually between pH 7 and 8. Hard water will often have pH values between 7.5 and 8.5.

When doing this experiment, you might choose to compare water quality at two or more points on the same stream, in two or more different streams or lakes, or in a lake and a stream that runs into it.

OBJECTIVES

In this experiment, you will

- Use a Temperature Probe to measure water temperature.

- Use a Conductivity Probe to measure the conductivity of water.

- Use a pH Sensor to measure the pH of water.

- Make visual observations at the test sites.

- Compare water quality.

MATERIALS

| LabPro interface | Vernier Conductivity Probe |
| --- | --- |
| Palm handheld | Vernier pH Sensor |
| Data Pro program | colorless plastic bottle |
| Temperature Probe | plastic cup or beaker |

Figure 1

PROCEDURE

Part I  Measuring Temperature

1.	Plug the Temperature Probe into Channel 1 of the LabPro interface. Connect the handheld to the LabPro using the interface cable. Firmly press in the cable ends.

2.	Press the power button on the handheld to turn it on. To start Data Pro, tap the Data Pro icon on the Applications screen. Choose New from the Data Pro menu or tap  to reset the program.

3.	Set up the handheld and interface for the Temperature Probe.

- If the handheld displays the correct TEMP(C) in CH 1, proceed directly to Step 4. If it does not, continue with this step to set up your sensor manually.

- On the Main screen, tap .

- Tap  to select Channel 1.

- Press the Scroll buttons on the handheld to scroll through the list of sensors.

- Select the correct Temperature Probe (in °C) from the list of sensors

- Tap  to return to the Main screen.

4.	Fill the plastic bottle with water taken below the water surface at a point one meter from the shore. CAUTION: Take all necessary precautions to ensure your safety!

5.	Measure the water temperature.

- Place the Temperature Probe into the sample.

- Gently move the probe in the water and note the temperature reading displayed at the top of the main screen.

- When the temperature stops changing, record the reading in your data table.

Part II  Measuring Conductivity

6.	Plug the Conductivity Probe into Channel 1 of the LabPro interface. Set the Conductivity Probe on the 0-2000 µS/cm position.

7.	Set up the handheld and interface for the Conductivity Probe.

- Choose New from the Data Pro menu or tap  to reset the program.

- If the handheld displays CONDUCT(MICS) in CH 1, proceed directly to Step 8. If it does not, continue with this step to set up your sensor manually.

- On the Main screen, tap .

- Tap  to select Channel 1.

- Press the Scroll buttons on the handheld to scroll through the list of sensors.

- Select CONDUCT 2000(MICS) from the list of sensors.

- Tap  to return to the Main screen.

8.	Measure the conductivity of the same water sample.

- Place the Conductivity Probe into the water. Briefly swirl the probe in the water.

- Once the conductivity is steady, record the reading in your data table.

Part III  Measuring pH

9.	Plug the pH Sensor into Channel 1 of the interface.

10.	Set up the handheld and interface for the pH Sensor.

- Choose New from the Data Pro menu or tap  to reset the program.

- If the handheld displays pH in CH 1, proceed directly to Step 11. If it does not, continue with this step to set up your sensor manually.

- On the Main screen, tap .

- Tap  to select Channel 1.

- Press the Scroll buttons on the handheld to scroll through the list of sensors.

- Select PH from the list of sensors.

- Tap  to return to the Main screen.

11.	Measure the pH of the same water sample.

- Remove the pH Sensor from its protective container and store the container safely aside.

- Rinse the pH Sensor using a plastic cup or beaker and water from the site.

- Place the pH Sensor into the water sample. Briefly swirl the pH Sensor in the water.

- Once the pH is steady, record the reading. Return the pH Sensor to its container.

12.	Note and record the clarity (clearness) of the water sample.

13.	Make and record other observations (concerning algae, plants, water “critters,” animals, flow rate, etc.) related to water quality at the site.

14.	Repeat Steps 1-13 at one or more other sites.

DATA AND OBSERVATIONS

| Site |  |  |
| --- | --- | --- |
| Temperature (°C) |  |  |
| Conductivity (µS/cm) |  |  |
| pH |  |  |
| Clarity |  |  |
| Other observations |  |  |

PROCESSING THE DATA

1.	How did the water quality at the different sites compare?

2.	What differences did you find? Explain the differences.

3.	What similarities did you find? Explain the similarities.

4.	At which site was the water “best”? Explain why it was the best.

5.	What new ideas for testing water quality did you come up with while doing this experiment?

EXTENSIONS

1.	Test water quality at the same site at different times.

2.	Test the effects of rain or snow on water quality.

3.	Test water quality at sites upstream and downstream from a wastewater treatment plant.