Velocity

Speed and velocity are rates. They tell us how much distance is covered in a unit of time. Velocity can be expressed by the formula

v = d/t

where v = velocity or speed (in m/s), d = distance traveled (in meters), and t = time (in sec). In this experiment, you will study the velocity of a car after it is released from different points on a ramp. A Motion Detector will be used to measure velocity.

OBJECTIVES

In this experiment, you will

- Measure velocity using a Motion Detector.

- Calculate average velocities.

- Determine the relationship between velocity and release point.

MATERIALS

| LabPro interface | several books |
| --- | --- |
| Palm handheld | meter stick |
| Data Pro program | masking tape |
| Vernier Motion Detector | car |
| 1.8 m board | small index card |

Figure 1

PROCEDURE

1.	Prepare the track.

- Set up a ramp on books as shown in Figure 1. The high end of the ramp should be 45 cm above the floor.

- Place a meter stick down the center of the ramp. The 0 cm mark on the meter stick should be at the very bottom of the ramp. Tape the meter stick to the ramp in two places. The meter stick will serve as a guide rail for your car.

- Fasten the Motion Detector at the top and center of the ramp as shown in Figure 1.

2.	Use tape to fasten a small index card to the back of the car you will be using. The card acts as a reflective surface for the Motion Detector.

3.	Plug the Motion Detector into the DIG/SONIC 1 port of the LabPro interface. Connect the handheld to the LabPro using the interface cable. Firmly press in the cable ends.

4.	Press the power button on the handheld to turn it on. To start Data Pro, tap the Data Pro icon on the Applications screen. Choose New from the Data Pro menu or tap  to reset the program.

5.	Set up the handheld and interface for data collection.

- On the Main screen, tap .

- On the Setup screen, tap .

- Enter “0.2” as the time between samples in seconds, using the onscreen keyboard (tap “123”) or using the Graffiti writing area.

- Enter “15” as the number of samples. (Data collection will last 3 seconds.)

- Tap  twice to return to the Main screen.

6.	Collect data.

- Place your car on the ramp with its front wheels at the 40 cm line.

- Tap . Release the car after you hear a sound coming from the Motion Detector.

7.	Determine the velocity of the car.

- After data collection stops, tap the y-axis label and choose DIG/SON 1:VEL(m/s).

- To examine the data pairs on the graph, tap  or any data point. As you move the examine line, the velocity and time values of each data point are displayed to the right of the graph.

- The highest point on the graph corresponds to the highest velocity of the car. Record this highest velocity in the data table. Round to the nearest 0.01 m/s. (In the example to the right, the highest velocity is 1.52 m/s.).

8.	Repeat Steps 6 and 7 two more times.

9.	Repeat Steps 6-8 at distances of 70 cm and 100 cm.

DATA

|  | Velocity (m/s) |  |  |  |
| --- | --- | --- | --- | --- |
| Release point | Trial 1 | Trial 2 | Trial 3 | Average |
| 40 cm |  |  |  |  |
| 70 cm |  |  |  |  |
| 100 cm |  |  |  |  |

PROCESSING THE DATA

1.	Calculate the average velocity for each release point. Show your work here. Record the results in the data table.

2.	Make a graph of your data. Plot Release Point (in cm) on the horizontal or x-axis and Average Velocity (in m/s) on the vertical or y-axis.

3.	What happens to the velocity of the car as you release it from higher points?

4.	How does friction affect the velocity of your car?

5.	How could you improve your car to make it roll faster?

6.	Describe one way you could change the ramp to make the car go faster, without changing the release points.

EXTENSIONS

1.	Repeat the experiment with ramps of different heights.

2.	Design, build, and test a different car.