Crash Dummies

What happens to people in automobile collisions is important information for car manufacturers. Crash dummies are often used in collision studies. Velocity is one of the most important factors in such collision studies. In this experiment, you will study the relationship between car velocity and the distance a “crash dummy” is thrown during a collision. A Motion Detector will be used to measure car velocity. You will use a ruler to measure the distance a crash dummy is thrown.

OBJECTIVES

In this experiment, you will

- Measure velocity.

- Measure distances an action figure is thrown.

- Calculate averages.

- Graph the results.

- Make conclusions from your results.

MATERIALS

| LabPro interface | meter stick |
| --- | --- |
| Palm handheld | masking tape |
| Data Pro program | car |
| Vernier Motion Detector | 2 small index cards |
| 1.8 m board | small action figure (10-12 cm tall) |
| several books | ruler |

Figure 1

Figure 2

PROCEDURE

1.	Prepare the ramp.

- Set up a ramp on books as shown in Figure 1. The high end of the ramp should be 45 cm above the floor.

- Place a meter stick down the center of the ramp. The 0 cm mark on the meter stick should be at the very bottom of the ramp. Tape the meter stick to the ramp in two places. The meter stick will serve as a guide rail for your car.

- Use tape to mark a distance 30 cm from the bottom end of the ramp. Place the bound edge of a heavy book on the 30 cm line where it will be struck head on by a car rolling off the ramp.

- Fasten the Motion Detector at the top and center of the ramp as shown in Figure 1.

2.	Prepare a car for data collection.

- Tape a card to the back of a car. This card serves as a reflector for the Motion Detector.

- If your car does not have a seat for a crash dummy, fold another card in half. Fold each side up as shown in Figure 2. Tape this folded card to the car to serve as a seat for the crash dummy.

- Seat the action figure (crash dummy) in the car.

3.	Plug the Motion Detector into the DIG/SONIC 1 port of the LabPro interface. Connect the handheld to the LabPro using the interface cable. Firmly press in the cable ends.

4.	Press the power button on the handheld to turn it on. To start Data Pro, tap the Data Pro icon on the Applications screen. Choose New from the Data Pro menu or tap  to reset the program.

5.	Set up the handheld and interface for data collection.

- On the Main screen, tap .

- On the Setup screen, tap .

- Enter “0.2” as the time between samples in seconds, using the onscreen keyboard (tap “123”) or using the Graffiti writing area.

- Enter “15” as the number of samples. (Data collection will last 3 seconds.)

- Tap  twice to return to the Main screen.

6.	Collect data.

- Place something heavy on the book to keep it from moving.

- Place your car on the ramp with its front wheels at the 40-cm line.

- Tap . Release the car after you hear a sound coming from the Motion Detector. Note: The car should crash into the book.

7.	Measure the distance the crash dummy was thrown (to the nearest cm). Record this value in Table 2. Measure from the seat of the car to the seat of the action figure.

8.	Determine the velocity of the car.

- After data collection stops, tap the y-axis label and choose DIG/SON 1:VEL(m/s).

To examine the data pairs on the graph, tap  or any data point. As you move the examine line, the velocity and time values of each data point are displayed to the right of the graph.

- The highest point on the graph corresponds to the highest velocity of the car. Record this highest velocity in the data table. Round to the nearest 0.01 m/s. (In the example to the right, the highest velocity is 1.52 m/s.).

9.	Repeat Steps 6-8 two more times.

10.	Repeat Steps 6-9 at distances of 60 cm and 80 cm.

DATA

| Table 1: Velocity (m/s) |  |  |  |
| --- | --- | --- | --- |
| Release point | 40 cm | 60 cm | 80 cm |
| Trial 1 |  |  |  |
| Trial 2 |  |  |  |
| Trial 3 |  |  |  |
| Average |  |  |  |

| Table 2: Distance Thrown (cm) |  |  |  |
| --- | --- | --- | --- |
| Release point | 40 cm | 60 cm | 80 cm |
| Trial 1 |  |  |  |
| Trial 2 |  |  |  |
| Trial 3 |  |  |  |
| Average |  |  |  |

#### PROCESSING THE DATA

1.	Calculate the average velocity for each release point. Show your work here. Record the results in the Table 1.

2.	Calculate the average distance that the crash dummy was thrown for each release point. Show your work here. Record the results in Table 2.

3.	Graph your results using the average values. Plot Velocity (in m/s) on the horizontal or x-axis and Distance Thrown (in cm) on the vertical or y-axis.

4.	What happened to the velocity of your car as it was released from higher release points?

5.	How did increased velocity affect the distance that your crash dummy was thrown?

6.	Do you think there would be more serious injuries in a high-speed or a low-speed collision? Explain.

7.	What is the purpose of seat belts?

EXTENSIONS

1.	Repeat the experiment with your car crashing into a moveable car instead of a book. Explain any differences in your results.

2.	Fasten the crash dummy into your car using a seat belt. Collect data while releasing the car from the 80 cm release point. Explain your results.